VI Међународни научни скуп Савремени материјали 2013 –  Бања Лука, 4-6 јул 2013.

VI International Scientific Conference Contemporary Materials 2013 – Banja Luka, 4-6 July 2013 

ЕПОКСИДНЕ КАТАФОРЕТСКЕ ПРЕВЛАКЕ НА ЧЕЛИКУ МОДИФИКОВАНОМ Zn-Mn ЛЕГУРАМА
М.В.Томић,1 М.М.Бучко,2 В.Б.Мишковић-Станковић,2 М.Г.Павловић,1 Ј.Б.Бајат2
1Универзитет у Источном Сарајеву, Технолошки факултет Зворник, Каракај бб, Република Српска, mtomicc@yahoo.com, 
[image: image1.emf]2Универзитет у Београду, Технолошко-металуршки факултет, Карнегијева 4, 11120 Београд, Србија, jela@tmf.bg.ac.rs, 
Заштитна својства органских превлака зависе од бројних фактора при чему модификација метлног супстрата има важну улогу. Модификација челика на бази легура цинка са елементима тријаде гвожђа, пре наношења органске превлаке, се врло много испитује последњих десетак година. Међутим, модификација Zn-Mn легуром до сада још није анализирана. У овом раду је испитиван утицај модификације површине челика електрохемијски таложеном превлаком Zn-Mn легуре на адхезију и корозиону стабилност танке епоксидне превлаке без пигмента. Епоксидна превлака је добијена катафоретским таложењем из емулзије епоксидне смоле различитим временима таложења и при високом и ниском напону таложења. Корозиона стабилност заштитних система је испитивана спректросопијом електрохемијске импедансе (СЕИ) у 3% NaCl. Појава две временске константе на СЕИ дијаграмима заштитног система добијеног при ниском напону већ после 24h деловања агенса корозије указује на његова лошија својства, која су потврђена и одређивањем адхезије. На основу свих резултата одређени су оптимално време и напон таложења.
Кључне ријечи: Zn-Mn легура, катафоретско таложење, епоксидна превлака, корозија 

EPOXY CATAPHORETIC COATINGS ON STEEL MODIFIED BY Zn-Mn ALLOYS
M.V.Tomić,1 M.M.Bučko,2 V.B.Mišković-Stanković,2 M.G.Pavlović,1 J.B.Bajat2
1Faculty of Technology Zvornik, University of Eastern Sarajevo, Republic of Serpska
2University of Belgrade, Faculty of Technology and Metallurgy, Karnegijeva 4, Belgrade, Serbia, 
There are many factors that influence the corrosion stability of organic coatings and surface pretreatments play an important role. Pretreatments based on zinc coatings alloyed with iron-group elements have been widely investigated during a last decade. However, steel surface modification with Zn-Mn coating prior to deposition of organic coating has not been investigated so far. In this work an attempt has been made to determine the influence of steel surface pretreatments with electrodeposited Zn-Mn alloy on the adhesion and corrosion stability of thin, nonpigmented epoxy coating. Epoxy coating was deposited cataphoretically from epoxy emulsion during different times and at both low and high voltage. The corrosion stability of protective systems was analyzed by electrochemical impedance (EIS) in 3% NaCl solution. The appearance of two time constant EIS plots, after only 24 h of exposure to corrosion agens, in protective system based on epoxy coating deposited at low voltage indicates its poor corrosion stability. This behaviour was confirmed by adhesion measurements. Based on all results тhe optimum deposition time and voltage were determined. 
Key words: Zn-Mn alloy, cataphoretic deposition, epoxy coating, corrosion
