# ИНТЕРАКЦИЈА ПОТОМАКА РАДОНА СА ЧВРСТИМ ТРАГ ДЕТЕКТОРИМА И УНУТАРСОБНИМ СТАКЛЕНИМ ПОВРШИНАМА У ФУНКЦИЈИ РЕТРОСПЕКТИВНЕ ДОЗИМЕТРИЈЕ

# А. Јанићијевић1. Д. Мирјанић2, М. Срећковић3, М. Давидовић4,

1) Технолошко-металуршки факултет, Београд, Србија, 2) Академија наука и умјетности Републике Српске,  3) Електротехнички факултет, Београд,  4) Институт ''Гоша'', Београд,

Сажетак:	Овај рад приказује неке аспекте коришћења чврстих нуклеарних траг детектора  за мерења радона у стамнбеним објектима. Приказани су и анализирани геометријски модели формирања таргова у материјалима коришћених детектора.  Посебно је разматрана употреба ових  детектора за процену изложености становништва радоновим потомцима такозваним  дуговременским пасивним методама за период од 1 месеца до 1 године. Међутим, такви подаци не дају најадекватније вредности  за укупну  изложеност и процену ризика излагњу радону и његовим потомцима. Посматрано у дужем временском интервалу појединци могу да мењају стамбено окружење, на пример реновирањем објеката. Зато мерење  радона у одређеном тренутку обично не даје адекватан податак о изложености неке особе радоновим потомцима у прошлости. Да би се решили овакви проблеми користе се резултати ретроспективне дозиметрије који се базирају на мерењу уграђених радонових потомака α - емитера (210Po) у стаклене  површине (прозорске или стаклене површине делова намештаја).

Кључне речи:  Чврсти нуклеарни траг детектори; нуклеарни трагови; LR 115; ретроспективна дозиметрија radona;

INTERACTION OF RADON PROGENY WITH SOLID TRACK DETECTORS AND ROOM GLASS SURFACES IN A FUNCTION OF  RETROSPECTIVE DOSIMETRY

# A. Janićijević1, D. Mirjanić2, M. Srećkovic3, M. Davidović4

1) Faculty of Technology and Metallurgy,  Belgrade, Serbia, 2) Academy of Sciences and Arts of the Republic Srpska, Banja Luka, Republik of Srpska, 3) Faculty of Electrical Engineering, Belgrade, 4) Institute ’’Goša’’, Beograd,

Abstract:	This paper reviews some aspects of solid-state nuclear track detectors (SSNTDs) and their applikation in the radon in residential buildings. Several geometrical models for the track growth given in the literature are described and compared. Applications of SSNTDs for radon and progeny measurements are discussed. Particular attention has been given to methods of long term passive measurements of radon progeny with SSNTDs. Long-term passive methods based on track detectors can give radon concentrations in dwellings for periods usually from 1 month to 1 year. The obtained radon concentrations prezent the contemporary values. However, to assess the total radon exposure of a person, such measurements might not  be satisfactory. For examle, a person could heve taken some actions that have significantly changed the radon levels, such as home renovation. Radon measurements performed at a particular time might not be able to represent the exposure of a person in the past. However , reconstruction of irradiation history for an individual is usualy very dificult, if not impossible. To solve this problem, the so-called retrospective radon measurement was proposed, wich are based on measurements of implanted radon progeny  α – emitters (210Po)  in glas surfaces or volumens of some detectors.

Keywords: Solid-state nuclear track detectors (SSNTD); Nuclear tracks; LR 115; Retrospective radon dosimetry;