Proučavanje energetske efikasnosti modula od monokristalnog silicijuma

Akademik Dragoljub Mirjanić 1, Siniša Maksimović2,  Darko Divnić1,

1 Akademija nauka i umjetnosti Republike Srpske, Bana Lazarevica 1Banja Luka, Republika Srpska, mirjanicd@gmail.com, divnic@gmail.com

2Elektroprenos BiH, Operativno područje Banja Luka, Republika Srpska, sinisa.maksimovic@elprenos.com

Apstrakt: U radu je prikazan sistem sa 8 solarnih panela od monokristalnog silicijuma i dodatna oprema za monitoring i mjerenja rada solarne elektrane, koji su postavljeni na zgradu Akademije nauka i umjetnosti Republike Srpske u Banjoj Luci. Pomoću U-I Analizatora za fotovoltaike PV-KLA i Mini PV-KLA vršena su mjerenja karakteristika solarnih modula. Meterološki parametri (temperatura, brzina vjetra, vlažnost vazduha,intenzitet sunčevog zračenja) su mjereni pomoću automatske meterološke stanice Davis Vantage Pro-USA. Prikazani su rezultati upoređivanja teorijskih i eksperimentalno dobijenih energetskih efikasnosti navedenih panela u odnosu na njihove karakteristike.

Ključne riječi: fotovoltaici, solarna energija, efikasnost, monokristalni silicijumski moduli

The study of the energy efficiency of monocrystalline silicon modules

Academician Dragoljub Mirjanić 1, Siniša Maksimović2,  Darko Divnić1,

Academy of Sciences and Arts of Republic of Srpska, 78 000 Banja Luka, Bana Lazarevića 1, Republic of Srpska Srpska, e-mail:mirjanicd@gmail.com, divnic@gmail.com

2Elektroprenos BiH Operational area of ​​Banja Luka, Republic of Srpska, sinisa.maksimovic@elprenos.com

Abstract: This paper presents a system with 8 solar panels with monocrystalline silicon and accessories for the monitoring and measurement of solar power plants, which are placed on the roof of the Academy of Sciences and Arts of the Republic of Srpska in Banja Luka. With UI analyzer for photovoltaic PV-KLA and Mini PV-KLA were carried out measurement of the solar modules. Meteorological parameters (temperature, wind speed, humidity, intensity of solar radiation) were measured using an automatic meteorological station Davis Vantage Pro-USA. This paper showing us the results of comparing between the theoretical obtained energy efficiency and experimentally obtained energy efficiency, above  mentioned  panels in relation to their characteristics.

Keywords: Photovoltaic, solar energy, efficiency, Monocrystalline Silicon Modules