LOGO WILL BE HERE

EFFECT OF BULK MODULUS OF FLUID ON THE DYNAMIC BEHAVIOR OF THE HYDRAULIC SYSTEM

Darko Knežević, Saša Laloš, Mitar Jocanović, Velibor Karanović

Summary: The bulk modulus is the physical characteristics of the fluid, which mainly affects dynamic characteristics of the hydraulic system. The value of bulk modulus depends on the state of fluid (pressure and temperature) and thermodynamic change of state (isothermal or adiabatic).In the most commonly used hydraulic fluid - mineral hydraulic oil - a significant impact on changing the bulk modulus has the presence of undissolved air in the oil.

This paper analyses the effect bulk modulus has on the elastic properties of the hydraulic fluid and the dynamic behavior of the hydraulic system.

Key words: bulk modulus of fluid, pressure, temperature, stiffness of a hydraulic spring

INTRODUCTION

Liquids, like solids, have the property of elasticity. Under the influence of the pressure, liquid behaves like a solid body and any change in pressure corresponds to the appropriate change in volume. Compressibility is defined by the equation

, 	(1)

where K is bulk modulus and, for hydraulic oils without undissolved air, the value of bulk modulus depends on the pressure, temperature and thermodynamic change of state.

Elastic properties of hydraulic oil can be expressed by coefficient of elasticity [1]

,      	(2)

where:

h – height of column of fluid in the cylinder (the cylindrical container),

p - pressure,

- force of pressure.

- Fig. 1 Compressibility of fluid

The compressed oil in the cylinder fits the following mechanical model.

Fig. 2 Mechanical model

If we disregard the change of bulk modulus K, then coefficient of elasticity c is constant, and the deformation of the spring (reduction of the column height of hydraulic fluid) is given by

.      	(3)

ISOTHERMAL BULK MODULUS

Isothermal bulk modulus is characterized by the change in density (volume) of fluid under constant temperature

.      	(4)

Change of isothermal bulk modulus, in table 1 and figure 3, is given for a mineral hydraulic oil HM 46. [2]

As value of bulk modulus depends, almost exclusively, from chemical structure, this oil can be used as a representative oil for a wide range of mineral hydraulic oils that are used in practice. [1], [2]

- Table 1 Isothermal bulk modulus of mineral hydraulic oil

| - Absolute pressure - p [bar] | - KT [bar] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - 15oC | - 20 oC | - 30 oC | - 40 oC | - 50 oC | - 60 oC | - 80 oC | - 100 oC |
| - 1 | - 16953 | - 16514 | - 15693 | - 14940 | - 14246 | - 13605 | - 12460 | - 11465 |
| - 21 | - 17142 | - 16702 | - 15878 | - 15120 | - 14423 | - 13778 | - 12625 | - 11623 |
| - 41 | - 17332 | - 16890 | - 16062 | - 15301 | - 14600 | - 13952 | - 12791 | - 11781 |
| - 61 | - 17521 | - 17078 | - 16247 | - 15482 | - 14778 | - 14125 | - 12957 | - 11941 |
| - 81 | - 17711 | - 17266 | - 16431 | - 15664 | - 14956 | - 14300 | - 13125 | - 12101 |
| - 101 | - 17900 | - 17454 | - 16617 | - 15846 | - 15134 | - 14475 | - 13293 | - 12262 |
| - 141 | - 18280 | - 17831 | - 16988 | - 16211 | - 15493 | - 14827 | - 13631 | - 12587 |
| - 181 | - 18659 | - 18208 | - 17360 | - 16577 | - 15853 | - 15181 | - 13972 | - 12915 |
| - 201 | - 18849 | - 18397 | - 17546 | - 16761 | - 16034 | - 15359 | - 14144 | - 13081 |
| - 241 | - 19229 | - 18774 | - 17919 | - 17129 | - 16397 | - 15716 | - 14489 | - 13414 |
| - 281 | - 19608 | - 19152 | - 18294 | - 17499 | - 16762 | - 16076 | - 14837 | - 13750 |
| - 301 | - 19798 | - 19342 | - 18481 | - 17685 | - 16945 | - 16256 | - 15012 | - 13919 |
| - 401 | - 20748 | - 20289 | - 19422 | - 18616 | - 17866 | - 17166 | - 15897 | - 14777 |
| - 501 | - 21698 | - 21238 | - 20367 | - 19555 | - 18797 | - 18088 | - 16797 | - 15653 |

Fig 3 Isothermal bulk modulus

ADIABATIC BULK MODULUS

Adiabatic bulk modulus is characterized by the change in density (volume) of fluid in thermodynamic processes without exchange of heat

, 	(5)

. 	(6)

where:

cp – specific heat capacity at constant pressure,

cv – specific heat capacity at constant volume.

Unlike static or isothermal bulk modulus, which is significant for the relatively slow compression processes, adiabatic bulk modulus is relevant for the analysis of the speed of propagation of pressure, sudden unloading chambers under pressure and other processes where changes of state of oil occurs relatively quickly.

Table 2 Adiabatic bulk modulus of mineral hydraulic oil [2]

| - Absolute pressure - p [bar] | - KS [bar] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - 15oC | - 20 oC | - 30 oC | - 40 oC | - 50 oC | - 60 oC | - 80 oC | - 100 oC |
| - 1 | - 19806 | - 19262 | - 18248 | - 17323 | - 16477 | - 15699 | - 14319 | - 13131 |
| - 21 | - 20003 | - 19457 | - 18441 | - 17513 | - 16663 | - 15882 | - 14494 | - 13300 |
| - 41 | - 20199 | - 19653 | - 18634 | - 17703 | - 16851 | - 16066 | - 14671 | - 13469 |
| - 61 | - 20396 | - 19848 | - 18827 | - 17894 | - 17038 | - 16250 | - 14848 | - 13639 |
| - 81 | - 20592 | - 20044 | - 19021 | - 18085 | - 17226 | - 16435 | - 15027 | - 13811 |
| - 101 | - 20789 | - 20240 | - 19214 | - 18276 | - 17415 | - 16620 | - 15205 | - 13983 |
| - 121 | - 20985 | - 20435 | - 19408 | - 18468 | - 17604 | - 16807 | - 15385 | - 14156 |
| - 161 | - 21378 | - 20827 | - 19797 | - 18852 | - 17983 | - 17180 | - 15747 | - 14504 |
| - 201 | - 21770 | - 21219 | - 20187 | - 19230 | - 18365 | - 17557 | - 16112 | - 14857 |
| - 241 | - 22163 | - 21611 | - 20577 | - 19626 | - 18748 | - 17935 | - 16480 | - 15213 |
| - 261 | - 22359 | - 21807 | - 20772 | - 19820 | - 18940 | - 18126 | - 16665 | - 15392 |
| - 301 | - 22752 | - 22200 | - 21164 | - 20209 | - 19326 | - 18508 | - 17037 | - 15754 |
| - 401 | - 23734 | - 23184 | - 22148 | - 21189 | - 20300 | - 19473 | - 17981 | - 16672 |
| - 501 | - 24718 | - 24171 | - 23137 | - 22178 | - 21285 | - 20452 | - 18942 | - 17612 |

Fig. 4 Adiabatic bulk modulus

DISSOLUTION OF AIR IN MINERAL OIL

All liquids have ability to absorb air, which in dissolved state does not impact liquid mechanical property. Volume of gas which could be absorbed by the liquid, till the liquid reaches its saturation, depends on pressure, temperature, type of gas and type of liquid. Bunsen`s coefficient is used to rate gas dissolving ability in liquid. The Bunsen`s coefficient is defined as volume of gas on atmospheric pressure and temperature of 0 oC, which could be dissolved in unit volume of liquid.

In mineral oils which are used in hydraulic systems, the value of air Bunsen`s coefficient is. [3]

At atmospheric pressure and temperature 0-100 oC the volume of air which could be dissolved in oil is practically constant, so the Bunsen`s coefficient is constant too.

The volume of air that can be dissolved in some additional volume of mineral oil until pressure of 300 bar is:

, 	(7)

where:

Vg - volume of dissolved air,

Vu - volume of oil and

p - absolute pressure of air in oil.

During operation of hydraulic system, oil usually contains 3-6% of undissolved air (at atmospheric pressure). By certain conditions, which depend on construction and exploitation conditions of system, the volume of undissolved air in oil can rise up to 15%. [4]

BULK MODULUS OF MIXTURE OF MINERAL HYDRAULIC OIL AND AIR

Increased working environment elasticity, caused by presence of undissolved air in oil, will increase with the increase of total volume of undissolved air, independent of air bubble dimension.

The analysis will be done using hydraulic cylinder example with remark that the obtained results for equivalent stiffness do not depend on pressure chamber shape.

The bulk modulus of mixture of oil and air, at given pressure, could be determined by using analogy with mechanical model, separating the whole volume of pressurized cylinder chamber in two parts: part filled with oil and part filled with undissolved air. As bulk modulus is used for description of elasticity properties of working environment, in the case of mixture of oil and air, equivalent stiffness of working environment can be determined as equivalent stiffness of two springs conceded in series.

From equation (2) can be seen that stiffness (bulk modulus) of oil is

,	 (8)

where:

Ku - bulk modulus of oil,

A - cylinder cross-section area and

hu - part of the cylinder height that is filled with oil,

while stiffness (bulk modulus) of air is

,	 (9)

where:

Kv - bulk modulus of air and

hv - part of the cylinder height that is filled with air.

Fig. 5  Mechanical model for calculating the equivalent stiffness of the mixture of oil and air

Equivalent stiffness is given by equation:

,                 	 (10)

As

, 	(11)

We get,

,	 (12)

respectively

, 	(13)

where:

KE - bulk modulus of working environment (mixture of oil and air,

Vu – cylinder volume part that is filled with oil,

Vv - cylinder volume part that is filled with air and

V – total cylinder volume.

- 5.1. EQUIVALENT BULK MODULUS OF MIXTURE OF MINERAL HYDRAULIC OIL AND AIR UNDER ADIABATIC CHANGE OF STATE

If the change of air state is adiabatic, then the volume of undissolved air in oil, depending on pressure, is given by equation

.	 (14)

Equivalent bulk modulus of oil and air mixture for adiabatic change of state of oil at compression and adiabatic change of state of undissolved air in oil, is then

. 	(15)

Fig. 6 Equivalent adiabatic bulk modulus: a) without the presence of undissolved air; b) 0,5 % undissolved air; c) 1 % undissolved air; d) 3 % undissolved air; e) 5 % undissolved air

Fig. 7 Equivalent adiabatic bulk modulus depending on the pressure and temperature in the presence of 1% undissolved air in the oil

Adiabatic compression of air bubbles that are in oil leads to their significant warming. The connection between temperature and pressure, at adiabatic change of state of air is given by equation:

,	 (16)

where:

TA1 i p1 - absolute temperature and pressure at the beginning of the process and

TA2 i p2 - absolute temperature and pressure at the end of the process.

It can be seen that at adiabatic compression of air bubbles that are in hydraulic oil, their temperature can increase significantly (e.g. at adiabatic compression of air with starting temperature of 30 oC, from atmospheric to 70 bar pressure, temperature of air increases to approximately 750 oC). This causes forming of flammable oil-air aerosols, which in case of ignition leads to very high oil temperature. The indicator for such processes is black colored hydraulic oil (similar to used oil from internal combustion engines) and burning smell.

CONCLUSION

- Working environment elasticity has significant influence on work efficiency and dynamic behavior of hydraulic system.

- Presence of undissolved air in hydraulic oil, especially in region of low pressure (under 20 bar) leads to significant change of bulk modulus.

- Undissolved air in hydraulic fluid causes action delay in hydraulic components (hydro motors and cylinders) and decreases their self-oscillation stability. It is similar to mass-spring mechanical system, where stiffness (elasticity) of working environment determines behavior of working components oscillations.

- Working environment elasticity is very important if sudden depressurizing of hydraulic cylinder chambers occurs (especially for cylinders with large working volume). If that is the case and if the depressurizing process is uncontrolled, the hydraulic hammer will occur.

REFERENCES

- Knežević, D. (2007). Influence Clearances Geometry in Hydraulic Components of Automation Control on Efficiency of Hydraulic Systems. Doctoral Thesis (2007), Faculty of Technical Science Novi Sad.

- Frauenstein, M. (1979). Manual for fluids Under Pressure for Designers, Mobil Oil.

- Savić, V. (1991).  Oil Hydraulics, Ikos, Zenica.

- Bašta, T.M. (1990). Mechanical Hydraulics, Faculty of Mechanical Engineering, Beograd.

PhD Darko Knežević, Banja Luka, Faculty of Mechanical Engineering, darko.knezevic@unibl.rs

MsC Saša Laloš, Banja Luka, Faculty of Mechanical Engineering, sasa.lalos@unibl.rs

PhD Mitar Jocanović, Novi Sad, Faculty of Technical Science, mitarjocanovic@gmail.com

MsC Velibor Karanović, Novi Sad, Faculty of Technical Science, velja_82@uns.ac.rs