КАРАКТЕРИЗАЦИЈА СТРУКТУРЕ, САСТАВА И РАСПОДЈЕЛЕ СЕКВЕНЦИ ПОЛИ(УРЕТАН-УРЕА-СИЛОКСАНА) 2D NMR И КВАНТИТАТИВНОМ 13C NMR СПЕКТРОСКОПИЈОМ

М. Балабан1, В. Антић2, Ј. Ђонлагић3

1Универзитет у Бањој Луци, Природно-математички факултет, Младена Стојановића 2, 78000 Бања Лука, e-mail: balabanm@pmfbl.org, 2Универзитет у Београду, Пољопривредни факултет, Немањина 6, 11000 Београд, e-mail: vantic@agrif.bg.ac.rs, 3Универзитет у Београду, Технолошко-металуршки факултет, Карнегијева 4, 11000 Београд, e-mail: jasna@tmf.bg.ac.rs

Сажетак: Хемијска структура и састав сегментираних поли(уретан-уреа-силоксанских) кополимера, PUUS, са тврдим сегментима на бази 4,4'–метилендифенилдиизоцијаната (MDI) и етилендиамина (ED), и меким сегментима на бази α,ω–дихидроксипропил-поли(диметилсилоксана) (PDMS) проучавана је 1H и 13C NMR спектроскопијом, као и примјеном више различитих 2D–NMR корелационих метода. Просјечна дужина секвенци тврдих и меких сегмената, као и расподјела секвенци тврдих сегмената у серији PUUS кополимера, добијена је коришћењем квантитативне 13C NMR спектроскопије.

Кључне ријечи: сегментирани поли(уретан-уреа-силоксани), расподјела сегмената, квантитативна 13C NMR спектроскопија

CHARACTERIZATION OF STRUCTURE, COMPOSITION AND SEQUENCE DISTRIBUTION OF POLY(URETHANE-UREA-SILOXANE)S BY 2D NMR AND QUANTITATIVE 13C NMR SPECTROSCOPY

M. Balaban1, V. Antić2, J. Đonlagić3

1University of Banja Luka, Faculty of Science, Mladena Stojanovića 2, 78000 Banja Luka, e-mail: balabanm@pmfbl.org, 2University of Belgrade, Faculty of Agriculture, Nemanjina 6, 11000 Belgrade, Serbia, e-mail: vantic@agrif.bg.ac.rs, 3University of Belgrade, Faculty of Technology and Metallurgy, Карнегијева 4, 11000 Belgrade, e-mail: jasna@tmf.bg.ac.rs

Abstract: The chemical structure and composition of segmented poly(urethane-urea-siloxane) copolymers, PUUS, based on 4,4'-methylene diphenyl diisocyanate-ethylene diamine (MDI-ED) hard segments and α,ω–dihydroxypropyl-poly(dimethylsiloxane) (PDMS) soft segment were investigated by 1H and 13C NMR spectroscopy and using various 2D–NMR correlation methods. The mean sequence length of the soft and hard segments, as well as the sequence distribution of the hard segments in the PUUS series, were obtained by quantitative  13C NMR spectroscopy.

Keywords: segmented poly(urethane-urea-siloxane)s, sequence distribution, quantitative 13C NMR spectroscopy