Symposium: Biomaterials and Nanomedicine - poster presentation

Prijavljen pod brojem #3988  http://ekonferencije.com/sr/rad/detection-of-protein-deposits-/3988

Detection of Protein Deposits on Soft Contact Lenses Using Near Infrared Spectroscopy

Ivan Djuricic, Jovana Šakota – Rosić, Ivana Mileusnić,   Jelena Munćan, Boris Kosić, Šarac Dušan, Lidija Matija

Nanolab, Biomedical Engineering, Faculty of Mechanical Engineering, University of Belgrade, Serbia

Immediately after being placed on the eye, contact lenses are coated with a protein layer and most proteins attach strongly to the material, with typically less than 50% being removed by conventional care regimens. The deposition of certain proteins to contact lenses has shown to increase the risk of microbial cell attachment to the lens material, and is also associated with inflammatory complications [1].

The purpose of this study was the classification between the groups of worn and new -unused soft contact lenses of different materials and produced by different manufacturers, using near infrared spectra of the contact lenses. Application of multivariate analysis on the spectral data showed that it is possible to distinguish the two groups, based on the absorbance of water molecules involved in protein hydration (1425nm, [2]). It can be concluded that NIR spectroscopy could be used for studying of protein depositions on soft contact lenses in their natural biological conditions, and furthermore prove insight into the mechanism of adsorption.

This exploratory study showed the potential of near infrared spectroscopy for non-invasive, chemical free detection of protein deposits on contact lenses, which could in future allow for simple and rapid characterization of contact lens materials from the aspects of protein adsorption and thus provide simple solution for the choice of optimal biomaterial formula for soft contact lenses development and production.

- Luensmann, D., & Jones, L. (2012). Protein deposition on contact lenses: The past, the present, and the future. Contact Lens and Anterior Eye

- Tsenkova, R. (2009). Introduction: Aquaphotomics: dynamic spectroscopy of aqueous and biological systems describes peculiarities of water. Journal of Near Infrared Spectroscopy, 17(1), 303.

-------------------------------------------------------------₰-----------------------------------------------------------

Glavni autor i prezenter: Ivan Djuricic, djuricicivan@gmail.com

E mail adrese ostalih autora:

Jovana Šakota Rosić  jovana.sakota@gmail.com ,

Ivana Mileusnić  ivana1318@gmail.com,

Jelena Munćan  jelena_muncan@yahoo.co.uk ,

Boris Kosić  bkosic@hotmail.com ,

Dušan Šarac saracdusan1986@gmail.com ,

Lidija Matija lmatija@mas.bg.ac.rs