Symposium: Water - poster presentation

Prijavljen pod brojem   #3989  http://ekonferencije.com/sr/rad/ftir-atr-study-of-aqueous-solu/3989

FTIR – ATR study of aqueous solutions of human serum albumin in physiological concentrations

Ivan Djuricic, Jovana Šakota – Rosić, Ivana Mileusnić, Jelena Munćan, , Dusan Sarac, Boris Kosic, Lidija Matija

Nanolab, Biomedical Engineering, Faculty of Mechanical Engineering, University of Belgrade, Serbia

Fourier transformed infrared spectroscopy of human serum albumin (HSA) in the region of 650-4000cm-1  was subjected for developing reagentless measurement of HSA.

The infrared spectra were subjected to partial least squares (PLS) regression to obtain calibration models predicting the concentrations of the HSA. The calibration in the concentration of 1 - 6.5mg/dl has yielded a correlation coefficient of 0.99 and a standard error of prediction of 0.11mg/dl with latent variable of six. These result suggests that the infrared spectra of aqueous solutions of HSA  and its calibration model developed using PLS regression can extract up effectively the information about albumin contained in water.

These results provide a base for development of reagentless method of measurement of HSA in human serum and also in vivo HSA measurement.

-------------------------------------------------------------------------₰-----------------------------------------------------------

Glavni autor i prezenter: Ivan Djuricic, djuricicivan@gmail.com

E mail adrese ostalih autora:

Jovana Šakota Rosić  jovana.sakota@gmail.com ,

Ivana Mileusnić  ivana1318@gmail.com,

Jelena Munćan  jelena_muncan@yahoo.co.uk ,

Boris Kosić  bkosic@hotmail.com ,

Dušan Šarac saracdusan1986@gmail.com ,

Lidija Matija lmatija@mas.bg.ac.rs