Savremeni materijali, Simpozijum B

Šarac D., Mitrović N., Tanasić I., Milošević M., Mišković Ž, Tihaček-Šojić Lj., Matija L,

Analysis of dental implants load transfer on the bone block model during axial loading using digital image correlation method.

Influence of implant on the surrounding bone under loading, is considered to be one of the most important characteristics when determining success of surgical treatment. Two different dental implants are placed into the polymethyl-methacrylate bone block model, in vertical position. Measured surface was then sprayed with layer of white paint, and stochastic pattern of black dots was added afterwards. Implant surface and block were fully bonded along the entire interface. Axial force was applied on dental implant with universal testing machine Tinius Olsen, up to 500 N. Deformations on the outer surface of the PMMA block were recorded using digital image correlation method (GOM). Displacements from the surface of the PMMA bone block models of different implants are analyzed and discussed.

Keywords: Dental Implant, Load transfer, Digital Image Correlation

Анализа трансфера оптерећења денталног импланта на модел кости приликом аксијалног оптерећења помоћу методе дигиталне корелације слика

Утицај импланта на околну кост током оптерећења се сматра једном од његових најважнијих карактеристика приликом одређивања успешности хируршког лечења. Два различита дентална импланта су постављена у блок од полиметил метакрилата(ПММА), у вертикалном положају. Мерна површина је обложена слојем беле боје, и стохастички шаблон црних тачкица је касније додат. Површина имплана и блока су чврсто везани дуж целе додирне површине. Аксијална сила је примењена на денталне импланте помоћу универзалне кидалице Тиниус Олсен, до 500 N. Деформације на спољној површини ПММА блока су мерене помоћу методе дигиталне корелације слика (ГОМ). Померања површине блока са различитим имплантима су анализирана и дискутована.

Кључне речи: Дентални имплант, Трансфер оптерећења, Дигитална корелација слика

Informacije o autorima:

- Šarac Dušan – saracdusan1986@gmail.com

Autor i prezenter, Beograd, Srbija

Ustanova: Mašinski fakultet Univerziteta u Beogradu

- Mitrović Nenad - nmitrovic@mas.bg.ac.rs

Beograd, Srbija

Ustanova: Mašinski fakultet Univerziteta u Beogradu

- Tanasić Ivan – doktorivan@hotmail.com

Beograd, Srbija

Ustanova: Stomatološki fakultet Univerziteta u Beogradu

- Milošević Miloš - mmilosevic@mas.bg.ac.rs

Beograd, Srbija

Ustanova: Univerzitet u Beogradu, Inovacioni centar mašinskog fakulteta

- Mišković Žarko - zmiskovic@mas.bg.ac.rs

Beograd, Srbija

Ustanova: Mašinski fakultet Univerziteta u Beogradu

- Tihaček-Šojić Ljiljana - prosthodontics.clinic@gmail.com

Beograd, Srbija

Ustanova: Stomatološki fakultet Univerziteta u Beogradu

- Matija Lidija - lmatija@mas.bg.ac.rs

Beograd, Srbija

Ustanova: Mašinski fakultet Univerziteta u Beogradu