Једнослојне полимерне фото ћелије. Могућности

Monolayer polymer photo cells. The opportunities

D. Dudić1,2, A.S. Luyt1,3

1 Department of Chemistry, University of the Free State, Private Bag X13, Phuthaditjhaba, 9866 South Africa

2 Vinča Institute of Nuclear Sciences, University of Belgrade, P.O. Box 522, 11001 Belgrade, Serbia

3 Current address: Center for Advanced Materials, Qatar University, PO Box 2713, Doha, Qatar

кључне речи: полимерна фото ћелија, енергија.

keywords: polymer photo cell, energy.

Претварање енергије сунца у електричну енергију је данас тема интензивних истраживања. Због познатих проблема насталих услед коришћења фосилних горива, коришћење обновљивих извора енергије је пожељно, док је у неким земљама коришћење ових извора енергије постало законска обавеза. Данас су комерцијално доступне силицијумске соларне ћелије са ценама од око 800 USD/kW (приближно 10 m2/kW) и ефикасношћу до 20 %. Основни проблем за ширу употребу ових извора енергије је још увек висока цена тако добијене електричне енергије у поређењу са ценама електричне енергије добијене из класичних електрана (термо, хидро и нуклеарне електране). Такође је остварен значајан напредак у области фото ћелија базираних на органским полимерима (OVP), енергетска ефикасност поменутих ћелија достиже вредност од 10 % и при томе њихова производна цена може бити значајно нижа од цене силицијумских ћелија. Постојећи резултати указују на изражене процесе старења полимерних фото ћелија и то је један од основних разлога њихове мале примене. Једнослојне полимерне фото ћелије карактерише једноставан дизајн и потенцијално веома ниска производна цена. Може се претпоставити да због поменутих особина једнослојне полимерне фото ћелије могу постати јефтина замена за широко распрострањене фото ћелије базиране на класичним полупроводницима. У овом раду су представљена постојећа публикована достигнућа у развоју једнослојних полимерних фото ћелија као и оригинална решења која су резултат истраживања аутора овог рада.

The conversion of solar energy into electricity is the focus of a large number of scientists. Because of the problems caused by the use of fossil fuels, the use of renewable and environmental friendly energy sources is desirable, while in some countries the use of a certain percentage of such energy sources becomes a legal obligation. Today there are commercially available silicon solar cells with prices ranging from 800 USD/kW (about 10 m2/kW) and efficiency in the range 10 – 20 %. The main obstacle to a wide use of these energy sources is the high price of such generated electric power in comparison with the costs of electricity from classical sources (hydro, thermal and nuclear power plants). Technological progress has been made in the development of polymer-based organic photovoltaic (OPV) solar cells, and their energy efficiency is approaching the value of 10 %, whereas the price per unit of output can be significantly lower than silicon solar cells. Shortcoming of OPV solar cells is their short life, and that is why they did not find greater application. Single-layer polymer photovoltaic characterized by simple design and potentially very low production cost. It can be assumed that because of these properties, the polymer monolayer photocell can become a cheap substitute for widespread photocell based on classic semiconductors. In this article, the existing published achievements in the development of monolayer polymer solar cell аre presentend, and also the original solutions that are the result of investigations conducted by authors.