CURRENT EFFICIENCY AND ENERGY CONSUMPTION DURING ELECTROCOAGULATION OF CYANIDE WASTEWATER

Borislav N. Malinovic*, Miomir G. Pavlovic1, Tijana Djuricic

University of Banja Luka, Faculty of Technology, 73 Stepe Stepanovica Street, 78000 Banja Luka, B&H;

1University of East Sarajevo, Faculty of Technology, Karakaj bb, 75400 Zvornik, Bosnia and Herzegovina; borislav.malinovic@unibl.rs

The research consisted in the application electrocoagulation process to remove cyanide from the simulated wastewater in a pilot plant, which is based on electrochemical flow reactor made of polyester in which has raised eight parallel electrodes. Results of electrochemical degradation of cyanide expressed through cyanide removal efficiency (Eu), current efficiency () and specific consumption of energy (WSP). At higher cyanide removal efficiency (Eu=47,43%),  current efficiency is lower and amount 25,96% (j=10 mA/cm2, (NaCl)=2 g/L, t=30 min), or 29,36% (Eu=27,09%, j=5 mA/cm2, (NaCl)=2 g/L, t=30 min), respectively. On the other hand, specific energy consumption increases with an increase in removal efficiency. For removal efficiency Eu=27,09%, it requires Wsp=35,50 kWh/kgCN energy (j=5 mA/cm2, (NaCl)=2 g/L). Also, specific energy consumption is higher at higher current density (j=10 mA/cm2)  and at the same concentration of supporting electrolyte (NaCl)=2 g/L was achieved Eu=47,43% with a specific energy consumption Wsp=110,52 kWh/kg. Electrochemical removal of cyanide can be successfully performed using electrochemical coagulation processes, and further research may focus on increasing the current efficiency and reducing energy consumption, which would further enhance this treatment.

Key words: electrochemical treatment, electrochemical flow reactor, cyanide.