XI SAVJETOVANJE HEMIČARA, TEHNOLOGA I EKOLOGA REPUBLIKE SRPSKE

XI Conference of  Chemists, Technologists and Environmentalists of Republic of Srpska


BIOLOŠKO UKLANJANJE AMONIJUM JONA IZ VODE ZA PIĆE
*Dijana Drljača1, Božo Dalmacija2, Ljiljana Vukić1, Slobodanka Zorić3
1Tehnološki fakultet Univerziteta u Banjoj Luci, Banja Luka, BiH  e-mail: dijana.drljaca@unibl.rs
2Prirodno-matematički fakultet Univerziteta u Novom Sadu, Departman za hemiju, biohemiju i zaštitu životne sredine,  Novi Sad, Republika Srbija
3 a.d. “Vodovod” Banja Luka, BiH
Biološki procesi uklanjanja amonijum jona iz vode za piće, poput biofiltracije, zauzimaju sve više pažnje, s obzirom da se primjenom ovih procesa mogu izbjeći toksični nusproizvodi dezinfekcije, ali i problemi koji se pojavljuju u distribucionom sistemu.
Istraživanja su sprovedena na poluindustrijskom postrojenju ("Pilot") u a.d. „Vodovod“ Banja Luka projektovanom za ispitivanje i optimizaciju procesa pripreme vode za piće iz površinske vode (rijeka Vrbas).
Ciklus istraživanja biološkog uklanjanja amonijum jona na filterima proveden je u ljetnom, jesenjem i zimskom periodu pri hidrauličkom opterećenju sirove vode od 1,2 – 2 m3/h.
Biološka filtracije na poluindustrijskom postrojenju, praćena je na jedno- i dvo-medijumskim filterima uz promjenu brzine filtracije i temperature ulazne vode. Pri brzini filtracije od 3,3 m/h nije postignuta potpuna nitrifikacija. Nepotpunost nitrifikacije ogleda se u nepotpunosti njene druge faze – nitratifikacije, dok je proces nitritifikacije konstantan na oba filtera (pijesak; pijesak+antracit), za sve primijenjene inicijalne koncentracije amonijum jona (γ = 0,6-1,3 mg NH4-N/L). Pri brzini filtracije od 0,8 m/h postiže se potpuna nitrifikacija čak i pri snižavanju temperature sa 11,1 na 8,8°C. 
Ključne riječi: pilot postrojenje, voda za piće, nitrifikacija, brzina filtracije
BIOLOGICAL REMOVAL AMMONIUM IONS 
FROM DRINKING WATER
*Dijana Drljača1, Božo Dalmacija2, Ljiljana Vukić1, Slobodanka Zorić3
1 University of Banja Luka, Faculty of technology, Banja Luka, BiH e-mail: dijana.drljaca@unibl.rs
2 University of Novi Sad, Faculty of sciences, Department for  Chemistry, Biochemistry and Environmental  Protection, Novi Sad, The Republic of Serbia;

3 a.d. „Vodovod” Banja Luka, BiH
Biological processes for ammonium ion removal from drinking water, such as a bio-filtration, take up more and more attention, due to the application of these processes may avoid the toxic by-products of disinfection, but also the problems that occur in the distribution system.

The study was conducted at the pilot plant facility ("Pilot") in the a.d. "Vodovod" Banja Luka designed for testing and optimization of the process of preparation of drinking water from surface water (river Vrbas).

Cycle Research biological removal of ammonium ions on the filters was carried out in summer, autumn and winter period when the hydraulic load of raw water from 1,2 – 2,0 m3/h.

Biological filtration pilot plant facility, accompanied by the one- and two-mode of the filters to the change in the speed of filtration and temperature of incoming water. In filtration rate of 3,3 m/h is not achieved complete nitrification. Incompleteness nitrification is reflected in the incompleteness of its second phase - nitratifikation, while the process nitritifikation constant on both filters (sand, sand + anthracite), for all the applied initial concentration of ammonium ions (γ = 0,6 – 1,3 mg NH4-N/L). In the filtration rate of 0,8 m/h provides the complete nitrification even at lower temperature from 11,1 to 8,8°C.

Key words: pilot plant, drinking water, nitrification, filtration rate
