Међународни научни скуп Савремени материјали –  Бања Лука.

International Scientific Conference Contemporary Materials – Banja Luka 

THE USE OF POLYLACTIDE BASED MATERIALS FOR 3D PRINTING
Ljiljana Tanasić1, Borislav Simendić2, Nevena Vukić3, Vesna Teofilović.3, 
Tamara Erceg3
[image: image1.emf]1High Agricultural School of Vocational Studies, Šabac, Serbia..
2Higher Education Technical School of Professional Studies, Novi Sad, Serbia
3University of Novi Sad, Faculty of Technology, Serbia
ljiljana3101@open.telekom.rs ugt.seminarski@gmail.com  nevenavukic@gmail.com  vesnavele@gmail.com ercegt7@gmail.com
Abstract: 3D printing is a new, rapidly developing technology which has the potential to revolutionise fabrication of certain high value, complex products. It is the preparation of an object by progressive addition of material layers. The main disadvantages of additive manufacturing are the limited material types that can be processed in this way and the fact that the material properties are often inferior to those used in more conventional preparation methods. Polylactide (PLA) has been becoming a very important polymer in the 3D printing processing, considering its low toxicity and its better environmental friendliness, compared to all the petroleum-based plastics. PLA belongs to the family of aliphatic polyesters derived from renewable resources, such as corn starch, sugar cane, tapioca roots or even potato starch and has a narrow processing window due to its tendency to undergo thermal decomposition in the molten state.
Keywords: PLA, printing, aliphatic polyesters, bio-based polymer 
UPOTREBA MATERIJALA NA OSNOVU PLA

 U 3D ŠTAMPARSTVU
Ljiljana Tanasić1, Borislav Simendić2, Nevena Vukić3, Vesna Teofilović.3, 
Tamara Erceg3
1Visoka poljoprivredna škola strukovnih studija,  Šabac, Srbija.
2.Visoka tehnička škola strukovnih studija u Novom Sadu, Srbija
3Univerzitet u Novom Sadu, Tehnološki fakultet, Srbija
Sažetak: 3D štampanje je nova tehnologija koja se ubrzano razvija i ima potencijal da unapredi proizvodnju veoma složenih proizvoda. Podrazumeva dobijanje predmeta postepenim dodavanjem materijala u slojevima. Glavni nedostaci ovog tehnološkog postupka su ograničen broj materijala koji mogu biti korišćeni na ovaj način. Pored toga svojstva ovih materijala su često sasvim drugačija u odnosu na materijale koji se koriste u drugim konvencionalnim postupcima proizvodnje. Poli(laktid) (PLA) je postao vrlo značajan polimer u procesu 3D štampe, zbog svoje niske toksičnosti. Sa ekološke strane je prihvatljiviji za životnu sredinu u odnosu na sve polimere na osnovu sirovina koje se dobijaju iz nafte. PLA pripada grupi alifatičnih poliestara, dobija se iz obnovljivih sirovina kao što su kukuruzni skrob, šećerna trska, koren tapioke, čak i krompirovog skroba i ima uzak temperaturni opseg preradljivosti usled toga što podleže termičkoj degradaciji u istopljenom stanju.

Ključne reči: PLA, štamparstvo, alifatični poliestri, polimeri na osnovi obnovljivih sirovina

