Jasmina Bogdanović-Jovanović, Živojin Stamenković, Živan Spasić, Jelena Petrović, Miloš Kocić
Numerical Investigation of Cavitating Flow in Venturi Nozzle

[image: image1.jpg]


[image: image40.png]vapor.Volume Fraction
Plane 2

9.891e-001

7.418e-001

4.945e-001

2.473e-001

1.000e-015

0 0050 0.100 (m)
0025 0075




Numerical investigation of cavitating flow in Venturi nozzle 
Jasmina Bogdanović-Jovanović
, Živojin Stamenković
, Živan Spasić3, Jelena Petrović4, Miloš Kocić5
Summary: Cavitation problems occurs in water flow where the local pressure reaches saturated vapour pressure, due to the sudden increase of local velocity values, forming bubbles and different forms of flow discontinuity. Water flow in the Venturi nozzle is the typical example of potentially cavitation flow. Due to the large importance of Venturi nozzles in engineering practice, it is important to determine their cavitation characteristics, such a cavitation number and loss coefficient of the nozzle. In case we are unable to obtain cavitation characteristics of the nozzle experimentally, there is the possibility to conduct numerical simulations of the flow in the Venturi nozzle. The objective of this study is to present the possibilities of obtaining cavitation characteristics of Venturi nozzles by applying numerical simulations, and to determine the optimal numerical modelling of flow in the case of Venturi nozzle.

Key words: Venturi nozzle, numerical simulation, cavitation, cavitation number, Venturi loss coefficient.
1. INTRODUCTION
Venturi effect is based on the sudden local flow velocity increase, due to the principle of mass continuity in the specific geometry of the nozzle, which leads to the local static pressure decrease, according to the low of mechanical energy conservation. The most common use of a Venturi tubes is in the measuring devices, such as Venturi meters used for measuring the volumetric flow rate of the liquids. The application of the Venturi tubes in the process industry and the industry in general is huge and significant, therefore the determination of their operating (and cavitation) characteristics are considerably important. Operating characteristic of Venturi nozzle depend on flow characteristics and geometry of the nozzle. The formation and development of cavitation in the Venturi strongly affect the operation of hydraulic system in which it is installed, usually as the limiting factor. 

There are several phases in the development of cavitation in a convergent-divergent nozzle such as Venturi nozzles [7,8], from cavitation inception through partial cavitation, full cavitation and supercavitation. Generally with increasing local flow velocity a pressure decrease in the throat of Venturi nozzle until it reaches the value of saturation pressure when vapour bubbles firstly appear in the water (so called cavitation inception). The bubbles grow and move toward the region with higher pressure, where they collapse suddenly, usually in the area near solid surfaces, causing the undesired phenomena of cavitation erosion. With a further reduction of pressure, bubbles grow and increase their number, accumulating into pockets of vapour, affecting the entire flow domain downstream. Unexpectedly, the further development of cavitation reduces the surface damage of the Venturi nozzle. In the regime of supercavitation a larger scale bubbles are formed in the central part of the nozzle, forming a kind of bubble spray. The air bubbles collapses downstream in the central part of the Venturi nozzles, away from the wall surfaces. Many attempts have been made to fully clarify the mechanisms of cavitation generally [11] and in a Venturi nozzle [1]. However, considering such a nonstationary phenomenon with numerous influential factors, this topic is still of interest to the scientific community.

In previous research a lot of attention is devoted to experimental measurements in convergent-divergent nozzles [2,3]. Using sophisticated high speed cameras is enabled the visualization of cavitation and obtaining cavitation patterns for all cavitation regimes [2].

In order to overcome the lack of laboratory space and equipment, there were many attempts to conduct numerical simulation of fluid flow in Venturi nozzles, with emphasis on two-phase flow, which occurs due to cavitation [4,5,9,12].

2. Cavitation parameters 

For characterization of the cavitation usually is used so called cavitation number, represented by the formula:


[image: image2.wmf](

)

-

s=

r

2v

2

th

pp

v/2






(1)

where p2 is downstream pressure, pv is the pressure of the vapour phase and vth is the velocity in the throat of the Venturi nozzle. 


Occasionally, a cavitation index Kc is used, mostly for studying cavitation in valves [13], or sometimes its reciprocated value (:


[image: image3.wmf](

)

(

)

e==--

c1v12

1/Kpp/pp




(2)
There are many empirical formulas for obtaining a saturated vapour pressure, depending on the temperature. For temperatures above 00C, some of well-known formulas are:

-
Tetens' formula:
    
[image: image4.wmf]×

+

=×

17,27t

t237,3

p0,61078e

 



(3)

-
Arden Buck formula:

[image: image5.wmf]æöæö

-

ç÷ç÷

+

èøèø

=×

tt

18,678

234,5257,14t

p0,61121e

,


(4)

where p – pressure [kPa], t – temperature [oC].

For water temperature t=25oC, a value of saturated vapour pressure is pv≈3168Pa. 
Energy dissipation in Venturi nozzle can be observed using Venturi loss coefficient defined by the formula:


[image: image6.wmf](

)

-

z=

r

12

2

th

pp

v/2





(5)

Dimensionless parameters allow more universal classification and and better overview of cavitation regimes. 
3. Geometry and discretization mesh of Venturi nozzle

Venturi nozzle is an example of convergent-divergent nozzle. A geometry of studied Venturi nozzle is shown in Fig.1.

 [image: image7.jpg]99

20

110





Fig. 1 Geometry of Venturi nozzle
Discretization mesh is made using software ICEM CFD 14.0. For the purpose of this investigation, there are few meshes created, in order to obtain the optimal one. For obtaining the optimal mesh, the grid independence test have been conducted.

3.3 Grid independence test 

To reduce the computational time and to ensure satisfactory accuracy of the results at the same time, different discretization meshes are tested, shown in Table I. The mesh number 2 in the Table I has been chosen, obtaining result error around 3% for the operating regime. The mesh is consists of 1056506 elements (565281 tetrahdra and 491225 wedges), shown in Fig.2.

Table 1 Grid independence test results (
[image: image8.wmf]&

m

=9 kg/s, k-turbulence model)
	Number of mesh elements
	y+
average
	Cavitation number σ
	Relative Error [%]

	~ 600000
	47.7
	0.253
	5.3

	~ 1280000
	30.9
	0.247
	3.0

	~ 2270000
	17.4
	0.245
	2.2



[image: image9]
Fig. 2 Discretization mesh of Venturi nozzle

For all three created meshes, the maximal value of y+ is around 100. For the chosen mesh with approximately 1280000 elements, in the central longitudinal plane the average value of y+=17.4, which is considered as a good quality mesh [10]. 
4. Numerical simultion of fluid flow in venturi nozzle

The two phase numerical simulations of water flow in the Venturi nozzle are conducted using Ansys CFX 14.0 for different flow regimes. The results are compared with eperimental results [2], quantitatively and qualitatively.
RANS numerical simulations are usually the first choice, considering the time and computational resources. In such highly non-stationary problems, it has been shown qualitatively good results obtained by RANS numerical simulations, eventhough the qualitatively the fluid flow pattern changes in time (the flow is unsteady). Four different turbulence models have been used (k-ε, k-ω, BSL Reynolds Stress and Shear Stress Transport - SST). The best results are obtained using SST model. Further on is determined functional dependence σ-ζ, shown in Fig.3. Rayleigh Plesset cavitation model is used for modeling dynamics of water bubbels in the cavitation regimes. As an advection scheme has been used the high-resolution scheme. 
[image: image10.png]1.4

e

02+~—F——"T—"—T——T—— 77T



[image: image11.png]1.4

0.6

0.4 1

0.2

0.1 0.2 0.3 0.4 0.5




[image: image12.png]1.4

T

ool

02+——F——7——— 11—



 [image: image13.png]1.4

T

ool

02+——F——7——— 11—




Fig. 3. σ-ζ diagrams for different mass flow rates: a)
[image: image14.wmf]=

&

m7 kg/s

, b)
[image: image15.wmf]=

&

m8 kg/s

,  c)
[image: image16.wmf]=

&

m9 kg/s

and d) 
[image: image17.wmf]=

&

m10 kg/s

.
Further, numerical investigations of ustationary nature of fluid flow in the Venturi nozzle have been conducted. Some qualitatively better results are obtained using URANS approach (i.e., transient numerical simulations). Detached Eddy Simulations, as the hybrid model, and, finally LES numerical simulations (Smagorinski) were used as well. Results obtained for the same flow regime (mass flow rate is 9 kg/s) and compared to experimental results [2]. All unsteady siulations obtained higly accurate values for cavitation number, Venturi loss coefficient and cavitation index. URANS and DES simulation results are very similar (maximal relative error 2% for the operating regime 
[image: image18.wmf]m

&

=9 kg/s), while LES simulation provides even more accurate results, with a relative error less then 1% for all coefficients. Unsteady numerical simulations offer a much better insight into the cavitation pattern. Fluid flow patterns evidently vary with numerical approaches. In Fig. 4 and Fig.5 is presented volume fraction of water vapor for different flow regimes.


[image: image19]
a)                                           b)


c)

Fig. 4. Vapor volume fraction in longitudinal section: a) URANS, b) DES, c) LES
[image: image20.png]vapor.Volume Fraction
Plane 3

1.0006+000

7.500e-001

5.000e-001

2.500e-001

0.000e+000

v



 [image: image21.png]yaporyolume Facton INNSYS

Plane 3
1.0006+000

7.500e-001
5.000e-001
2.500e-001

0.000e+000




 [image: image22.png]vapor.Volume Fraction
Plane 3

1.000e+000

7.500e-001

5.000e-001

2.500e-001

0.000e+000




 [image: image23.png]vapor.Volume Fraction
Plane 3

1.0006+000

7.500e-001

5.000e-001

2.500e-001

0.000e+000

[ 0015 003 (m)

00075 0.0225




Fig. 5. Vapor volume fraction in cross-section of the Venturi nozzle (z=0.35 m): a) RANS (SST), b) URANS, c) DES, d) LES
5. CONCLUSION 

Compared with available experimental results, numerical simulations of the water flow in the Venturi nozzle show quantitatively good results for various operating regimes (i.e. various water flow rates). Very accurate results of obtained pressure and velocity are provided even by RANS numerical simulations The average relative deviations of the obtained cavitation numbers depends on the Venturi flow rate, and it is only 2.8% for the flow rate of 10 kg/s, while the maximum average relative deviation of cavitation numbers is 8% for the flow rate of 7 kg/s. However RANS simulation cannot provide satisfactory results when it comes to the resulting flow field in cavitation regimes, particularly turbulent fluctuations in the output part of the Venturi nozzle, nor to predicte cavitation patterns. 

 In case one wants to examine the flow pattern in Venturi nozzle and to obtain a proper visualization of the cavitation regime transition, an unsteady numerical simulation of fluid flow is essential. In this paper URANS, DES and LES approach for one flow regime are compared, giving priority to the first method due to the shorter time required for the simulation. But to achieve greater accuracy, especially when it comes to obtaining fluctuating velocity field and cavitation patterns, priority must always be given to LES simulations.
REFERENCES 

Journal Papers:

[1] Charrière B., Decaix J., Goncalvès E., (2015). A comparative study of cavitation models in a Venturi flow, European Journal of Mechanics B/Fluids 49, 287–297.
[2] Rudolf P., Hudec M., Gríger M., Štefan D. (2014), Characterization of the cavitating flow in converging-diverging nozzle based on experimental investigation, EPJ Web of conferences, 67.
[3] Niedzwiedzka A., Sobieski V., (2016). Experimental investigation of caviting flows in a Venturi tube, Technical Sciences, 19(2) 151–164.
[4] Hari Vijay P., Subrahmanyama V., (2014). Cfd simulation on different geometries of venturimeter, IJRET: International Journal of Research in Engineering and Technology, Volume: 03, Issue:07, 456-463.
[5] Tamhankar N., Pandhare A., Joglekar A., Joglekar A., (2014). Experimental and CFD analysis of flow through venturimeter to determine the coefficient of discharge, International Journal of Latest Trends in Engineering and Technology (IJLTET), Vol. 3 Issue 4, 194-200.
[6] Koukouvinis P., Naseri H., Gavaises M., (2016). Performance of turbulence and cavitation models in prediction of incipient and developed cavitation, International J of Engine Research, 1-18.
Books:

[7] Dixon S. L., (1998). Fluid Mechanics and Thermodynamics of Turbomachinery, Elsevier Inc., 4th edition.
[8] Knapp R. T., Daily J. W., Hammitt F. G., (1970). Cavitation, McGraw-Hill.
[9] Frenander K., (2014). Optimization and Automated Parameter Study for Cavitating Multiphase Flows in Venturi - CFD Analysis, Master’s Thesis, Department of Applied Mechanics, Chalmers University of Technology.
[10] Ferziger, J. H., Peric, M.,(2002). Computational methods for fluid dynamics, third rev edition, Springer.
Proceedings Papers:

[11] Brennen C. E., (2011). An Introduction to Cavitation Fundamentals, invited lecture, Cavitation: Turbo-machinery & Medical Applications, WIMRC FORUM 2011, University of Warwick, UK.
[12] Kozak J., Rudolf P., Stefan D., Hudec M., Griger M., (2015). Analysis of pressure pulsation of cavitating flow in converging-diverging nozzle, 6th IAHR International Meeting of the Workgroup on Cavitation and Dynamic Problems in Hydraulic Machinery and Systems, Ljubljana, Slovenia.
WWW pages:

 [13] Samson A.G.,Cavitation in control valves, from http://www.samson.de
LOGO WILL BE HERE








�dr Jasmina Bogdanović-Jovanović,Niš,Serbia, Faculty of Mechanical Engineering (bminja@masfak.ni.ac.rs)


� dr Živojin Stamenković, Niš,Serbia, Faculty of Mechanical Engineering (zikas@masfak.ni.ac.rs) 


3 dr Živan Spasić, Niš,Serbia, Faculty of Mechanical Engineering (zivans@masfak.ni.ac.rs)


4 Jelena Petrović, Niš,Serbia, Faculty of Mechanical Engineering (coldfluids@gmail.com)


5 Miloš Kocić, Niš,Serbia, Faculty of Mechanical Engineering (kocicm@masfak.ni.ac.rs)





6
5

[image: image24.png]


[image: image25.png]vapor.Volume Fraction
Plane 2

9.822e-001

7.366e-001

4.911e-001

2.455e-001

1.000e-015



[image: image26.png]vapor.Volume Fraction
Plane 2

9.822e-001

7.366e-001

4.911e-001

2.455e-001

1.000e-015



[image: image27.png]vapor.Volume Fraction
Plane 2

9.822e-001

7.366e-001

4.911e-001

2.455e-001

1.000e-015



[image: image28.png]vapor.Volume Fraction
Plane 2

9.822e-001

7.366e-001

4.911e-001

2.455e-001

1.000e-015



[image: image29.png]vapor.Volume Fraction
Plane 2

9.822e-001

7.366e-001

4.911e-001

2.455e-001

1.000e-015



[image: image30.png]vapor.Volume Fraction
Plane 2

9.558e-001

7.168e-001

4.779e-001

2.389e-001

1.000e-015




[image: image31.png]vapor.Volume Fraction
Plane 2

9.558e-001

7.168e-001

4.779e-001

2.389e-001

1.000e-015




[image: image32.png]vapor.Volume Fraction
Plane 2

9.558e-001

7.168e-001

4.779e-001

2.389e-001

1.000e-015




[image: image33.png]vapor.Volume Fraction
Plane 2

9.558e-001

7.168e-001

4.779e-001

2.389e-001

1.000e-015




[image: image34.png]vapor.Volume Fraction
Plane 2

9.558e-001

7.168e-001

4.779e-001

2.389e-001

1.000e-015




[image: image35.png]vapor.Volume Fraction
Plane 2

9.891e-001

7.418e-001

4.945e-001

2.473e-001

1.000e-015

0 0050 0.100 (m)
0025 0075



[image: image36.png]vapor.Volume Fraction
Plane 2

9.891e-001

7.418e-001

4.945e-001

2.473e-001

1.000e-015

0 0050 0.100 (m)
0025 0075



[image: image37.png]vapor.Volume Fraction
Plane 2

9.891e-001

7.418e-001

4.945e-001

2.473e-001

1.000e-015

0 0050 0.100 (m)
0025 0075



[image: image38.png]vapor.Volume Fraction
Plane 2

9.891e-001

7.418e-001

4.945e-001

2.473e-001

1.000e-015

0 0050 0.100 (m)
0025 0075



[image: image39.png]vapor.Volume Fraction
Plane 2

9.891e-001

7.418e-001

4.945e-001

2.473e-001

1.000e-015

0 0050 0.100 (m)
0025 0075



_1552152942.unknown

_1552152968.unknown

_1552152996.unknown

_1552251244.unknown

_1552335252.unknown

_1552152979.unknown

_1552152958.unknown

_1552147804.unknown

_1552147819.unknown

_1552147831.unknown

_1552147751.unknown

