Miloš M. Kocić, Živojin M. Stamenković
MHD flow and heat transfer of micropolar fluid with induced magnetic field effects


[image: image1.jpg]



MHD flow and heat transfer of micropolar fluid with induced magnetic field effects

Miloš M. Kocić
, Živojin M. Stamenković1, Jelena D. Petrović1, Jasmina B. Bogdanović- Jovanovića1 and Milica D. Nikodijević

Summary: In this paper, the MHD flow and heat transfer of an incompressible electrically conducting micropolar fluid through a parallel plate channel is investigated. A uniform magnetic field was applied perpendicular to the flow and, due to the fluid motion, a induced magnetic field appears along the lines of motion. The upper and lower plates have been kept at the two constant different temperatures and the plates are electrically insulated. Partial differential equations governing the flow, heat transfer, micro rotation and magnetic field conservation are transformed to ordinary differential equations and solved under physically appropriate boundary conditions. Solutions obtained for the velocity, micro-rotation, temperature and induced magnetic field in function of the coupling parameter and the spin-gradient viscosity parameter are presented graphically.

Key words: Micropolar fluid, heat transfer, MHD flow, magnetic Reynolds number.
1. INTRODUCTION 
The requirements of modern technology have stimulated the interest in fluid flow studies, which involve the interaction of several phenomena. One of these phenomena is certainly viscous flow of electrically conducting micropolar fluid in the presence of a magnetic field. The theory of thermo-micropolar fluids has been developed by Eringen [1], taking into account the effect of micro-elements of fluids on both the kinematics and conduction of heat. The concept of micropolar fluid is introduced in an attempt to explain the behavior of a certain fluid containing polymeric additives and naturally occurring fluids such as the phenomenon of the flow of colloidal fluids, real fluid with suspensions, liquid crystals and animal blood, etc...

The flow and heat transfer of a viscous incompressible electrically conducting fluid between two infinite parallel insulating plates has been studied by many researchers [2,3] due to its important applications in the further development of MHD technology. The MHD devices for liquid metals attracted the attention of metallurgist [4]. It was shown that the effect of magnetic field could be very helpful in the modernization of technological processes. The increasing interest in the study of MHD phenomena is also related to the development of fusion reactors where plasma is confined by a strong magnetic field [5]. Many exciting innovations were put forth in the areas of MHD propulsion [6], remote energy deposition for drag reduction [7], MHD control of flow and heat transfer in the boundary layer [8].

All the studies cited above are limited to classical Newtonian fluids. There are many fluids which are important from the industrial point of view, and display non-Newtonian behavior. 

Eringen [1] initiated the concept of micropolar fluids to characterize the suspensions of neutrally buoyant rigid particles in a viscous fluid. The micropolar fluids exhibit microrotational and microintertial effects and support body couple and couple stresses. It may be noted that micropolar fluids take care of the microrotation of fluid particles by means of an independent kinematic vector called microrotation vector.

According to the theory of micropolar fluids proposed by Eringen [1] it is possible to recover the inadequacy of Navier–Stokes theory to describe the correct behavior of some types of fluids with microstructure such as animal blood, muddy water, colloidal fluids, lubricants and chemical suspensions. In the mathematical theory of micropolar fluids there is, in general, six degrees of freedom, three for translation and three for microrotation of microelements. Extensive reviews of the theory and applications can be found in the review article [9] and in the book [10].

The MHD heat transfer of micropolar fluid can be divided in two parts. One contains problems in which the heating is an incidental by product of electromagnetic fields as in MHD generators etc, and the second consists of problems in which the primary use of electromagnetic fields is to control the heat transfer (Toshivo et al. [11]). Heat transfer in micropolar fluid flow in the presence of magnetic field has gained considerable attention in recent years because of its various applications in contemporary technology. These applications include liquid crystals [12], blood flow in lungs or in arteries [13] and etc.

Basic ideas and techniques for both steady and unsteady flow problems of Newtonian and non-Newtonian fluids are given by Ellahi [14]. The basic equations governing the flow of couple stress fluids are non-linear in nature and even of higher order than the Navier Stokes equations. Thus an exact solution of these equations is not easy to find. 

Keeping in view the wide area of practical importance of micropolar fluid flow and heat transfer as mentioned above and continuing our previous work [15], the objective of the present study is to investigate the MHD flow and heat transfer characteristics of  incompressible micropolar fluid in a parallel plate channel with induced magnetic field effects. 
2. MATHEMATICAL MODEL
The problem of laminar MHD flow and heat transfer of an incompressible electrically conducting micropolar fluid between parallel plates is considered. MHD channel flow analysis is usually performed assuming the fluid constant electrical conductivity and treating the problem as a one-dimensional one.

The physical model shown in Figure 1, consists of two infinite parallel plates extending in the x and z-direction. Fully developed flow takes place between parallel plates that are at a distance h, as shown in Figure 1. Electrically conductive fluid flows through the channel due to the constant pressure gradient and applied magnetic field.  A uniform magnetic field of the strength B is applied in the y direction, while the direction of flow, intensity of u, is in the x direction. Due the fluid motion magnetic field of the strength Bx is induced along the x axis (Reynolds magnetic number takes values around one). The upper and lower plate have been kept at the two constant temperatures 
[image: image2.wmf]1

T

 and, 
[image: image3.wmf]2

T

 respectively. 
[image: image4.jpg]Z

=
Micro- 2
Rotatonal .2 B p,H,G,k
Effects &

o1}

<

g

N





Fig. 1. Physical model and coordinate system

Described laminar MHD flow and heat transfer is mathematically presented with following nondimensional equations:


[image: image5.wmf](

)

22

2

110,

dudHadb

KK

dydyRmdy

w

++++=


(1)


[image: image6.wmf]2

2

20,

ddu

KK

dydy

w

w

G--=


(2)


[image: image7.wmf](

)

22

22

22

1PrPr0,

dduHadb

KEcEc

dydyRmdy

q

æöæö

+++=

ç÷ç÷

èøèø


(3)


[image: image8.wmf]2

2

1

0.

dbdu

Rmdydy

+=


(4)

The fluid and thermal boundary conditions, in nondimensional form, for this problem are represented by equations:


[image: image9.wmf]0,0,0,0for0;0,0,1,0for1.

ubyuby

wqwq

==========


(5)

The following transformations have been used to transform equations to nondimensional form:


[image: image10.wmf](

)

2

2

0

2

12012

,,,,,Pr,,.

p

x

p

c

TTBU

bKHaBhRmUhEc

TTBhkcTT

m

lgs

qsm

mmm

-

===G=====

--


(6)

After basic mathematical transformations from equations (1) and (2) the equation for microrotation is:


[image: image11.wmf]0,

iv

ab

www

-+=

"


(7)

where:


[image: image12.wmf]2

******

11

(-2),2,,,,.

111

KHaK

aRmBADbRmBDABCD

KKRmK

=-=====

+++G


(8)

The solution of equation (7), are giving three possible cases, but due to restriction of the number of pages, just the solution for the first case is given:


[image: image13.wmf](

)

(

)

(

)

(

)

(

)

11223344

expexpexpexp,

yCyCyCyCy

wdddd

=+++


(9)

[image: image14.wmf](

)

(

)

(

)

(

)

(

)

1112223334441

expexpexpexp,

uyCyCyCyCyD

dddd

=++++

DDDD


(10)


[image: image15.wmf](

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

(

)

11223344

613714823924

2

1111221331445101578

1Pr[exp2exp2exp2exp2

expexpexpexp

expexpexpexp()],

yKEcyyyy

yyyy

yyyyyDyD

qdddd

dddddddd

dddd

=-+++++

++++++++

++++++++

TTTT

TTTT

TTTTTTT


(11)



[image: image16.wmf](

)

(

)

(

)

(

)

(

)

1112223334444

*

1

[expexpexpexp].

byCyCyCyCyCyD

B

dddd

=-+++++

AAAA


(12)
2.1 RESULTS AND DISCUSION 
Obtained solutions for the velocity, micro-rotation, temperature and induced magnetic fields in function of  the coupling parameter and the spin-gradient viscosity parameter are generated graphically.
Figure 2 shows the effect of the spin-gradient viscosity parameter on velocity, which predicts that the velocity increases as the spin-gradient viscosity parameter decrease. This fact lead to the conclusion that increase of gyro-viscosity γ reduce the flow compared to the viscous fluid case. 

Micro-rotation in function of the spin-gradient viscosity parameter is shown in the Figure 3. Increasing of the spin-gradient viscosity parameter cause decrease in absolute values of micro-rotation.
[image: image17.jpg]Ha=2, K=5

1,00
——T=50
. - - -T=05
----- =0,005
0,75
=~ 0,50
0,25
0,00 T T T
0,0 0,5 1,0 1,5 2,0 2,5 3,0 3,5





[image: image18.jpg]1,00

0,75

~ 0,50+

0,25

0,00

-0,06

(4]

5 [ e
\
\
\
|
I
/
/
/
/-
I
<7
——
el &
% S T I [R—
. , .
0,00 0,02 0,04





	Fig. 2. Velocity profiles for different values of the spin-gradient viscosity parameter
	Fig. 3. Micro-rotation in function of the spin-gradient viscosity parameter


[image: image19.jpg]1,00

Ha=2, K=5

0,75

~ 0,50

0,25






[image: image20.jpg]Ha=2, K=5

1,00

0,75

=~ 0,50

0,25

0,00

T T T T

T T
0,0000 0,0005 0,0010

b

T T
-0,0010 -0,0005




	Fig. 4. Temperature in function of the spin-gradient viscosity parameter
	Fig. 5. Induced magnetic field in function of the spin-gradient viscosity parameter


On figure 4 is shown that increasing of the spin-gradient viscosity parameter cause decrease of dimensionless temperature over the entire width of the channel. Increase of the spin gradient viscosity reduce the amount of energy transformed in the fluid. As gyro-viscosity increases the dominant heat transfer mechanism is conduction.
Increasing of the spin-gradient viscosity parameter cause decrease in absolute values of induced magnetic field, which is shown on figure 5.

[image: image21.jpg]1,00

0,75

~ 0,50

0,25

0,00

Ha=2, I'=50

2,5 5,0

12,5





[image: image22.jpg]Ha=2, I=50

1,00 e
© N
\
\
0,75- '
/
7/
4
7
=z -
g
-~ 0,50- R ,r'/'
ek
> I
r'd
I
/
0,25 - /
|
ey — K=0,01
~L - --K=1I
AT IS R B LRERE K=5
004+—F—— 14— ==t L S
-0,00015 -0,00010 -0,00005 0,00000 0,00005 0,00010  0,00015

(0




	Fig. 6. Velocity profiles for different values of the coupling parameter
	Fig. 7. Micro-rotation for different values of the coupling parameter


Next figure, Fig. 6, shows the effect of the coupling parameter on velocity. From Figures 6 and 7 it can be observed that the increase in coupling parameter K decrease the velocity but increases the microrotation which means, as expected, that the resistance of fluid increases with the increase of K. The microrotation component 
[image: image23.wmf]w

 increases near the upper plate and decreases near the lower plate symmetrically with increasing K, showing a reverse rotation near boundaries. 

[image: image24.jpg]Ha=2, I'=50

1,00
—K=0,01
- - -K=]
----- K=5
0,75 -
1 /
rd
/
> 0,50 -
/7
7/
] /7
/7
7/
7
0,25 - .
7
p I’
0,00 === ; | ; |
0,0 0,5 1,0

1,5





[image: image25.jpg]1,00

0,25

0,00
-0,004

|
0,000 0,002

0,004




	Fig. 8. Dimensionless temperature in function of the coupling parameter
	Fig. 9. Induced magnetic field in function of the coupling parameter


Increasing of the coupling parameter cause decrease of dimensionless temperature over the entire width of the channel, which is shown on figure 5. Figure 9 shows, as it is expected, that increasing of coupling parameter cause decrease in absolute values of induced magnetic field.
3. CONCLUSION 

In this paper, the steady flow and heat transfer of an incompressible electrically conducting micropolar fluid through a parallel plate channel is investigated. The general equations that describe the discussed problem under the adopted assumptions are reduced to ordinary differential equations and closed-form solutions are obtained. Effects of the coupling parameter and the spin-gradient viscosity parameter on the heat and mass transfer have been analyzed. 
ACKNOWLEDGMENTS

This paper is supported by the Serbian Ministry of Sciences and Technological development (Project No. TR 35016; Research of MHD flow in the channels, around the bodies and application in the development of the MHD pump). 
REFERENCES (BOLD)

[1] Erigen, A. C. : Theory of micropolar fluids, J. Math. Mech. 16 (1966) 1-18.

[2] Blum, E. L., Zaks, M. V., Ivanov, U. I., Mikhailov, Yu. A. Heat and Mass transfer in the Presence of an Electromagnetic Field, (in Russian), Zinatne, (1967)  236 p. Riga.

[3] Attia, H.A., Kotb, N.A. MHD flow between two parallel plates with heat transfer. Acta Mechanica, Vol. 117 (1996), pp. 215-220.
[4] B. A. Sivak,V. G. Grachev,V. M. Parshin,A. D. Chertov, S. V. Zarubin, V. G. Fisenko, A. A. Solovev, MHD processes in the electromagnetic stirring of liquid metal in continuous section and bloom casters, Metallurgist, July (2009), Volume 53, Issue 7, pp 469-481

[5] Morley, N. B., Malang, S. and Kirillov, I. Thermo-fluid magnetohydrodynamic issues for liquid breeders, Fusion Science and Technology, (2005) Vol. 47, pp. 488-501

[6] M. Y. Abdollahzadeh Jamalabadi, Analytical study of magnetohydrodynamic propulsion stability, Journal of Marine Science and Application, (2014), Volume 13, Issue 3, pp 281-290

[7] V. Shatrov, G. Gerbeth, On Magnetohydrodynamic Drag Reduction And Its Efficiency, The 15th Riga and 6th PAMIR Conference on Fundamental and Applied MHD Instability and transition to turbulence in MHD, Riga, Latvia, 2005, pp. 149-152.

[8] Nikodijevic D.,  Stamenkovic Z., General characteristics of unsteady MHD temperature boundary layer, International journal of non-linear mechanics, (2015), vol.73, pp. 75-84

[9] T. Ariman, M.A. Turk, N.D. Sylvester, Microcontinuum field mechanics – a review, Int. J. Eng. Sci. 11 (1973) 905–929.

[10] G. Lukaszewicz, Micropolar Fluids, Theory and Application, Birkhauser, Basel, 1999.

[11] T. Toshivo, S. Ryoji, O. Hiroyuki, Magnetizing force modelled and numerically solved for natural convection of air in a cubic enclosure: effect of the direction of the magnetic field, International Journal of Heat and Mass Transfer, Vol. 45, Issue 2, pp. 267–277, 2002.

[12] A. Sengupta, U. Tkalec, M. Ravnik, J. M. Yeomans, C. Bahr, S. Herminghaus, Liquid Crystal Microfluidics for Tunable Flow Shaping,  Phys. Rev. Lett. 110, 048303 – Published 25 January 2013

[13] Kh. S. Mekheimer , M. A. El Kot, The micropolar fluid model for blood flow through a tapered artery with a stenosis, Acta Mechanica Sinica, 2008, Volume 24, Issue 6, pp 637-644

[14] Ellahi R., Steady and Unsteady Flow Problems for Newtonian and Non-Newtonian Fluids: Basics, Concepts, Methods. VDM Verlag, Germany, 2009.

[15] Kocić M.,Stamenković Ž,Petrović J.,Bogdanović-Jovanović J.,Nikodijević M., Heat transfer in micropolar fluid flow under the influence of magnetic field, Thermal Science, 2016, Volume 20, Issue supp. 5, pp. S1391 - S1404.
LOGO WILL BE HERE








� Faculty of Mechanical Engineering, University of Niš, A. Medvedeva 14, 18000 Niš, Serbia (milos.kocic@masfak.ni.ac.rs)


� Faculty of Occupational Safety, University of Niš, Čarnojevića 10a, 18000 Niš, Serbia








2
3

_1552990713.unknown

_1552991879.unknown

_1552992461.unknown

_1553503357.unknown

_1553505852.unknown

_1552991903.unknown

_1552991912.unknown

_1552991894.unknown

_1552990733.unknown

_1552991870.unknown

_1552990723.unknown

_1552896032.unknown

_1552990701.unknown

_1552906527.unknown

_1552896021.unknown

