Резултати експерименталног испитивања пећи на пелете према ЕN 14875

У раду су приказани резултати испитивања пећи мале снаге (8,5 kW) за сагоревање дрвених пелета која се користи за загревање домаћинстава. Експериментална испитивања су извршена на испитној инсталацији пројектованој и изведеној у Лабораторији за горива и сагоревање Машинског факултета Универзитета у Београду. Пројектовање и израда испитне инсталације као и целокупан поступак испитивања извршени су у складу са захтевима важећег Европског прописа ЕN 14785 за пећи ове врсте. Испитивања су обављена за три топлотна оптерећења пећи (минимално, средње и максимално) и са три врсте испитног горива. Као испитна горива коришћени су пелети различитог квалитета (произведени од буковог дрвета, али са различитим уделом коре и различитим технолошким параметрима процеса производње). Обрадом резултата испитивања у складу са захтевима стандарда омогућено је одређивање енергетских и еколошких карактеристика испитиване пећи и њена класификација према критеријумима дефинисаним стандардом ЕN 14785. 
Кључне речи: пелети, пећ мале снаге на пелете, енергетске и еколошке карактеристике, термотехника
Results of experimental investigation of pellet stove according to ЕN 14875

The results of investigation of small scale pellet stove (8.5 kW) for combustion of wood pellets used for household heating are presented in this paper. Experiments were performed on the test installation, designed and assembled in the Laboratory for Fuel and Combustion at the University of Belgrade, Faculty of Mechanical Engineering. Design and manufacturing of test installations, as well as the entire testing procedure were carried out in accordance with the requirements of the applicable European regulation EN 14785 for ovens of this type. Tests were conducted for three thermal loads of the stove  (minimum, medium and maximum) and three types of test fuel. Pellets of different quality (made ​​of  beech wood, but with different proportions of bark and various technological parameters of the production process) were used as test fuels. Recalculation of test results in accordance with the requirements of a standard provided determination of energy and environmental characteristics of the tested stove and its classification according to the criteria defined by standard EN 14785.
Keywords: pellets, small scale pellet stove, energy and environmental characteristics, HVAC

