ПРИЈАВА

Међународна научна конференција“Савремени материјали 2018“

КОМПАРАЦИЈА РАЗЛИЧИТИХ СРЕДСТАВА ЗА ДЕЗИНФЕКЦИЈУ ВОДЕ ЗА ПИЋЕ

Ђукић Вeљkо

1Паневропски универзитет АПЕИРОН Бања Лука, Факултет здравствених наука Бања Лука

Сажетак: Завршни поступак у припреми вода за пиће је дезинфекција. Дезинфекција представља деструкцију организама који су узрочници болести уз примјену различитих агенаса. Процес дезинфекције могуће је реализирати на различите начине што зависи од састава воде као и намјене воде. Код избора средства за дезинфекцију воде за пиће постоји неколико битних фактора, те је врло тешко извршити потпуно задовољавајући избор средства за дезинфекцију.

Код припреме вода за пиће данас се највише користи поступак са хлором. Поред предности при употреби хлора, све више јављају се недостаци, посебно код вода са високим садржајем хуминских киселина. Избјегавање ових проблема, могуће је ријешити коришћењем других средстава за дезинфекцију, као што су хлор диоксид, озон или УВ зраке.

У раду су приказана различита средства као и компарација тих средстава за дезинфекцију воде за пиће.  Према  учинку и својствима приоритет у примјени се даје озону. Међутим, због великих инвестиционих као и погонских трошкова употреба озона је ограничена само на погоне гдје  лош квалитет воде. Примјеном озона знатно се поправљају својства воде. Због непостојаности његовог резидуала потребна је додатна дезинфекција неким другим средством ради заштите дистрибутивне мреже од евентуално могућих загађења.

Кључне ријечи: вода за пиће, дезинфекција, озон.

COMPARISON OF DIFFERENT MEANS FOR DISINFECTION OF DRINKING WATER

Đukić Veljko

1Paneuropean University APEIRON Banja Luka, Health Science Faculty Banja Luka

Abstract: The final process in the preparation of drinking water  is disinfection. Disinfection is a destruction of organisms that cause disorders, using different agents. Disinfection process can be implemented in different ways depending on the composition of the water and uses water. When selecting a disinfectant for drinking water, there are several important factors, and it is very difficult to completely satisfactory selection of disinfectants means.

When preparing the drinking water today is mostly used process with chlorine. In addition to the advantages of using chlorine, there are more and more deficiencies, especially in water with a high content of humic acids. Avoiding these problems can be resolved by the use of other disinfectants means, such as chlorine dioxide, ozone or UV rays.

Different means are presented in this paper as well as a comparison of these means for disinfection of drinking water. According to the effect and properties of the priority in application is given ozone. However, due to large investment as well as operating costs, the use of ozone is limited to drives where poor water quality. The use of ozone significantly improves water propertieа. Due to the lack of stability of its residual, additional disinfection is required by some other means in order to protect the distribution network from possible pollution.

Key words: drinking water, disinfection, ozone..