Међународни научни скуп Савремени материјали –  Бања Лука.

International Scientific Conference Contemporary Materials – Banja Luka 

КОРОЗИОНА ЗАШТИТА МЕТАЛА ПРИМЕНОМ ИНХИБИТОРА


Ђенђи Ваштаг 
[image: image1.emf]Природно-математички факултет, Универзитет у Новом Саду, Трг Д. Обрадовића 3, Нови Сад, Србија

 djendji.vastag@dh.uns.ac.rs
Сажетак: Деградација материјала (сировина, производа или полупроизвода) услед корозије спада у један од кључних проблема савременог човечанства. Корозија као спонтана иреверзибилна реакција материјала са околином, доводи до промена својстава услед чега се њихова употребна вредност значајно смањује. Од свих употребних материјала корозијa метала и легура изазива највеће штете. За заштиту метала од корозије у привреди и у свакодневном животу примењују се различити видови заштите. Један од често примењених видова заштите је употреба корозионих инхибитора. У току дугогодишње праксе, инхибитори корозије метала су се временом мењали, прилагођавали потребама и захтевима друштва све са циљем постизања што ефикасније, дугорочније, безбедније, а у новије време и еколошки прикладније корозионе заштите метала.
Кључне ријечи: корозија метала, инхибитори корозије 
CORROSION PROTECTION OF METALS USING INHIBITORS
Abstract: Degradation of materials (raw materials, products or semi-finished products) due to corrosion is one of the key problems of modern humanity. Corrosion, as a spontaneous irreversible reaction of materials with the environment, leads to changes in properties, as a result of which their use value is significantly reduced. Of all the materials used, corrosion of metals and alloys causes the greatest damage. In the economy and in everyday life, different types of protection are used to protect metals from corrosion. One of the most commonly used types of protection is the use of corrosion inhibitors. During many years of practice, metal corrosion inhibitors have changed over time, adapted to the needs and requirements of society, all with the aim of achieving more efficient, long-term, safer and recently, more environmentally friendly protection of metals from corrosion.

Keywords: metal corrosion, corrosion inhibitors

