Међународни научни скуп Савремени материјали –  Бања Лука.

International Scientific Conference Contemporary Materials – Banja Luka 

ОСОБИНЕ АПСОРПЦИЈЕ МОНОХРОМАТСКЕ СВЕТЛОСТИ КРОЗ РАЗЛИЧИТЕ ТИПОВЕ УЉА



Бојан Павичар1
1Природнo-математички факултет, Универзитет у Новом Саду, Србија
Сажетак: Апсорпција светлости представља ефикасан метод за истраживање светлости и својстава средине кроз коју она пролази. На примеру проласка светлости кроз воду и различите врсте уља различитих таласних дужина, демонстрисано је ово својство. У овој студији, монохроматски ласери (405нм, 532нм, и 650нм) коришћени су као извори светлости. Путем анализе интензитета светлости која пролази кроз средину помоћу вакуумске фотоћелије, добијени подаци су конвертовани у облик електричних сигнала. У овој студији је такође истакнут Ламберт-Беров закон, који објашњава опадање интензитета светлости у зависности од дебљине течног слоја. Овај метод се може користити да се одреди дебљина уљног слоја у случају да дође до изливања уља у воду, што је значајно у контексту мерама заштите при евентуалном загађењу. Додатно, ова студија открива и појаву интензивне луминисценције при проласку светлости кроз уље.

Кључне речи: апсорпција, монохроматска светлост, уље, луминисценција

ABSORPTION CHARACTERISTICS OF MONOCHROMATIC LIGHT THROUGH VARIOUS OIL TYPES

               Abstract: Light absorption serves as a potent investigative tool for examining light's properties and the mediums through which it travels. Our study encompassed measuring light absorption across a spectrum of wavelengths, passing through diverse mediums including water and various oils. Utilizing monochromatic sources—violet (405nm), green (532nm), and red (650nm) lasers—we observed light transmission, capturing the optical signal and converting it into electronic data. Demonstrating the Lambert-Beer law, we identified diminishing light intensity relative to the absorbing medium's thickness. This technique proves invaluable for gauging oil layer depth in mixed water-oil mediums, such as during water contamination incidents, facilitating pollution quantification and guiding responsive measures. Beyond absorption, the presence of oil in water was vividly evidenced by intense luminescence phenomena.
Keywords: absorption, monochromatic light, oil, luminescence

