INFLUENCE OF DIFFERENT CONCENTRACIONS OF GOLD NANOPARTICLES ON SURFACE PROPERTIES AND ANTIMICROBIAL POTENTIAL OF DENTURE BASE ACRLYIC RESIN MATERIALS

Veselinović Valentina1, Trtić Nataša1, Adamović Tijana1, Dolić Olivera1, Arbutina Radmila1, Đeri Aleksandra1, Marin Saša1, Knežević Nataša1

- University of Banja Luka, Faculty of Medicine, Department of Dentistry

Purpose.

The gingivally supported denture base, made of conventional polymethyl methacrylate (PMMA), is susceptible to biofilm formation and the development of denture stomatitis. The purpose of this work is the production of PMMA enriched with gold nanoparticles (AuNPs) with improved antimicrobial properties.

Material and methods. AuNPs were added to denture base PMMA in concentration of 0,5% and 1%.  Heat-polymerized acrylic resin specimens of pure PMMA and nanomodified PMMA (PMMA/AuNPs) were prepared according to each test specification. Surface roughness and surface free energy were measuremed by Profilomethar, and hydrofobicity and contact angle were measured by Tensiometar. Antimicrobial activity of newly formed PMMA/AuNPs were evaluated for two different microbial streins (Candidae albicans and Staphylococcus aureus) in monomicrobial biofilm form. Colony forming units (CFUs), cell metabolic activity (MTT) and disc diffusion test for each tested strain were performed.

Results: The surface roughness, surface free energy increase in PMMA/AuNPs samples. There is no statisticaly differences between tested groups. Hidrofobicity and contact angle measurement shows differences compare to unmodified PMMA. PMMA/AuNPs showed a significant reduction in the monomicrobial biofilms of all tested species compare to control group  and it is in direct coleration with percentage of added AuNPs. The CFUs test of AuNPs modified PMMA samples did not show change compare to pure PMMA. Released AuNPs  did not recorded around the PMMA/AuNPs samples.

Conclusion: Incorporation of AuNPs into heat-polymerised denture base PMMA led to surface properties change. Antimicrobial activity of PMMA/AuNPs increase in corelation with percentage of AuNPs and it is significantly expressed even in a very small concentration of gold nanoparticles.

Key word: gold nanoparticles, denture base, acrylic resin, denture stomatitis