МАПА СУНЧЕВОГ ЗРАЧЕЊА ЗА ПЕТ ОПШТИНА СМЈЕШТЕНИХ У ЈУГОИСТОЧНОМ   И ИСТОЧНОМ ДИЈЕЛУ  РЕПУБЛИКЕ СРПСКЕ

THE MAP OF SOLAR RADIATION FOR FIVE MUNICIPALITIES LOCATED IN  THE SOUTHEASTERN AND EASTERN PART OF THE REPUBLIC OF SRPSKA

Душанка Марчетић1,  Драгољуб Мирјанић2,  Бојан Ковачевић1,  Дијaна Ђукић3, Светлана Пелемиш4,  Дарко Дивнић2

1Универзитет у Бањој Луци, Природно-математички факултет, Младена Стојановића 2, 78000 Бања Лука,  Босна и Херцеговина, e-mails: dusanka.marcetic@pmf.unibl.org, bojan.kovacevic@pmf.unibl.org

2Академија наука и умјетности  Републике Српске, Бана Лазаревића 1, 78000 Бања Лука, Босна и Херцеговина, e-mails: mirjanicd@gmail.com, divnic@gmail.com

3Универзитет у Бањој Луци,  Машински факултет, Војводе Степе Степановића 71, 78000 Бања  Лука, Босна и Херцеговина,  e-mail: dijana.djeordjic@mf.unibl.org

4Универзитет у Источном Сарајаеву, Технолошки факултет Зворник, Каракај 34А, 75400 Зворник, Босна и Херцеговина, e-mail: svetlana.pelemis@tfzv.ues.rs.ba

Сажетак: Процес  преласка на обновљиве изворе енергије услиједио је као ријешеност човјечанства  да ограничи  ефекте глобалног загријавања, узрокованог првенствено емисијом гасова стаклене баште. Соларна енергија је покретач већине процеса који се догађају на планети, и може се директно претварати у електричну струју кориштењем фотонапонских  технологија. Такав начин производње електричне  енергије се показао једним од најперспективнијих у циљу смањења емисије.  Од свих обновљивих извора енергије, Босна и Херцеговина највише користи хидро енергију, док је соларни потенцијал готово нетакнут. У овом раду,  направили смо мапу сунчевог зрачења  за пет општина смјештених у југоисточном и источном  дијелу Републике Српске. Мапа је добијена кориштењем PVGIS (фотонапонски географски информациони систем) базе података. За одабранa мјеста,  презентовали смо   средњу дневну ирадијацију, средњу мјесечну и годишњу ирадијацију,  директну нормалну ирадијацију,  глобалну ирадијацију под оптималним углом као и омјер дифузне и глобалне ирадијације те средњу температуру. Поред тога, дата је процјена  електричне енергије  која би се могла  добити из соларних електрана номиналне снаге 5kWp, инсталираних на крововима кућа или зграда. Ове процјене би могле бити од помоћи    становницима  при доношењу одлуке да   експлоатишу  соларну енергију као    самостални   произвођачи.

Abstract: The  world’s transition process  to renewable energy sources emerged as its  commitment to  limit  global warming, mainly  caused  by    the greenhouse gas emissions.   Solar energy  drives  most of   the processes on the planet, and it can be directly  converted  into electricity by using photovoltaic technologies. This way of electricity generation  proves to be one of the most promising in the achieving the   aforementioned  goal.  Among all renewables,  Bosnia and Herzegovina mostly exploits hydropower,  whereas its   solar potential   is almost untouched. In this paper,  we produce the  map  of solar radiation for five municipalities  located in the southeastern and eastern part of  the Republic of Srpska. The map is  obtained by using the Photovoltaic Geographical Information System (PVGIS) database.  For selected towns, we present average daily irradiation,  average monthly and yearly global horizontal irradiation,  direct normal irradiation,  global irradiation at  optimum angle as well as diffuse to global ratio and  average temperature.  Besides,  we give estimates of electricity that could be  generated by solar power plants of   5kWp nominal power, installed on the rooftops of households or  buildings. These estimates  may  guide householders  in their decisions  to harvest solar energy as   independent    energy producers.

Кључне ријечи:  обновљиви извори енергије, сунчево зрачење, фотонапонске технологије, соларне електране

Keywords: renewable energy sources, solar radiation,  photovoltaic technologies, solar power plants

- INTRODUCTION

Over about two centuries of rapid industrialization and massive exploitations of exhaustible natural resources, we have so immensely contaminated our planet that the life has become endangered. One of the steps  taken by humanity  to resolve this  situation is  the transition to renewable energy  sources. Solar energy is considered as  one of the most prosperous among all renewables. It is practically inexhaustible,   as  the Sun is expected  to  shine for  the  next  five billion years. Every second, from its surface, the Sun emits a vast amount of energy   in the form of electromagnetic radiation.  Only about 5x10-8 % of this, which approximates to 170 petawatts of  energy per second, reaches the outer layer of the Earth’s  atmosphere.  Some of this energy is  immediately reflected out  into space. Transmitted energy, in its way to the ground, is absorbed, scattered and reflected  by various  atmospheric components, mostly by the  clouds.    Depending  on  the geographical coordinates,  altitude, local landscape, part of the year and the day, etc.,  in average, approximately 150-300 W/m2 of solar energy per  second hits  one square meter of  the ground in an inhabited place [1]. In just  an hour and a half,  the Earth's surface  receives enough energy  to cover the annual demands of humanity [1,2]. 	Solar  energy can be collected  in a number of ways, but the most common  are solar thermal,  where the solar  energy is accumulated  as heat, which can further  be used to produce electricity, and photovoltaic (PV) solar,  where the solar energy is directly converted into electricity [3].  These technologies are safe, environmentally  friendly and ultimate  in order to achieve the world goal of net zero emissions.  They are  still not competitive to fossil fuel based energy or  to  other renewables, but it is expected  that   with the  accelerated development  of  PV technologies and infrastructure,  as well as  policy supports, they  will be the most abundant and cheapest way of electricity production in the next  decades.  This is  the part of  the world energy transition strategy  made to fulfill the Paris Climate Agreement’s 1.5°-degree goal by 2050 [4].

The world’s  leading countries trend toward renewable  energy utilization is certainly  reflected on small  and undeveloped countries such as Bosnia and Herzegovina (B&H), which slowly, but evidently is  preparing for this route.  B&H is  electricity exporter,  however,    more than 60%  of generated electricity  in 2022 is produced by  coal-fired power stations [5], while  the rest is   based on renewables, mostly  hydropower and a  small percentage  of  wind power.  Installed photovoltaic capacity  is around  100-150 MW currently, but  these numbers are increasing fast, since many solar power plants are  under  construction or  are planned  to be built.

Republic of Srpska (RS), one of the entities of B&H,    has  amended  its  law   of  2013 regarding renewables,  and the new Law on Renewable Energy Sources of RS  came into force in 2022 [6].   A year later, this law was complemented by the Rulebook on  Prosumers, which additionally specified prosumer’s  status, rights and obligations.  These  are the most  recent steps made by  the  RS Government in order to  instigate residents on   installation of solar panels on  their households.

In this paper we  estimate solar potential  of  five towns: Bileća, Ljubinje, Berkovići, Kalinovik and Rudo, located in the southeastern and eastern part of the Republic of Srpska.  We apply  Photovoltaic Geographical Information System (PVGIS)  utility [7]. PVGIS is free online application developed  in 2001 by the European Comission Joint Research Centre to assess  solar resources of Europe and  near  Mediterranean region.  Since then,   it has continuosly been improving [8,9].  Current version is PVGIS 5.2 and its data are collected by  METEOSAT satellite observations. By using PVGIS we  obtain average  daily, monthly and yearly irradiation on horizontal and  normal  plane as well as the plane  tilted at optimum angle. Diffuse to global ratio and average temperature are also  presented. Finnaly, we give PVGIS estimates for the electricity that could be generated by crystalline silicon 5 kWp solar modules  installed at  fixed optimum angle on the roofs of households and buildings. The  present study  is the  part of   Solar Atlas  of Republic of Srpska - a larger study regarding solar potential  assessment  of  the Republic of Srpska, and adds up to some previous research  on the topic [10-16].  With the changes in policies and subventions  offered by the Government of RS, we hope that  the massive installations of solar  panels (modules)  on existing  rooftops  would mark the upcoming period.   Thus, clean energy could be produced with no  additional degradation of land, whereas  reliance on thermal power plants could weaken.

- MAP OF SOLAR RADIATION FOR THE SELECTED REGION

Bosnia and Herzegovina is a country located in the southeastern part of Europe. Its climate varies from moderate continental  in  the northern Peri-Pannonian region, through transitional Mediterranean climate towards the south, to   Modified Mediterranean (Adriatic)  at  the  furthest south.  Mountainous   and Alpine climate prevails on altitudes higher than 1000 m. Bosnia and Herzegovina  has, on average,   about 1900 sun hours annually. In Herzegovina,  annual sunshine duration reaches up to 2300 hours [17], which makes it very favorable for PV utilization.  The   selected region  of the Republic of Srpska  for PV resource’s estimation  is

located  in the  southeastern and eastern  part of the RS.  Capitals of  the municipalities  of  the same name  are

presented in Figure 1. In Figure 2, for considered region, we present the map of  global  solar irradiation (GHI)

Figure 1. Geographical position of  selected towns  in the Republic  of  Srpska for which  we estimate solar radiation.

obtained by using  Global Solar Atlas [18]- free online application. GHI is solar  energy that  falls  on  one

square  meter of  a horizontal flat  surface during a  period of time.   As we see in Figure 2, daily average    GHI   varies over  the interval   3.2-4.4 kWh/m2, while the annual average   is in   the interval 1100-1600 kWh/m2.  It is also noticeable  that irradiation  increases towards the south.

2.1  Daily average irradiation profile

Bileća is the southernmost town of the considered  region. It  is located in the  karst  field    at  an altitude of around 470 m, and  has about 2200 sun hours per year.  Geographical coordinates  of Bileća are taken to be 42.874° (42° 52’ 26.4’’) north latitude and 18.429° (18° 25’ 44.4’’) east longitude.   For these coordinates, in Figure 3(a)-(d), we present average hourly values  of irradiance in January, April,  July and October, obtained from PVGIS   data collected  over   the period 2005-2020 with  the resolution of one hour.  Irradiance is the amount of energy that, in one second, hits  one square meter of a plane, i.e. it is a power per unit area.  Average global G(i), direct (beam) Gb(i) and diffuse Gd(i) irradiances on  a fixed horizontal plane are presented. Direct irradiance  strikes  the plane  directly from the Sun’s  disk,   whereas    diffuse irradiance  strikes the  plane   from all other  directions, after being scattered  by the atmosphere, mainly by the clouds.

Figure 2.  The map of GHI for selected region.

Figure 3. Hourly average global, direct and diffuse solar irradiance  distribution over  a day in January (a), April (b), July (c) and October (d) for Bileća.

They are   the components of  global irradiance, which represents   total irradiance o  the horizontal plane.  It is noticeable in  Figure 3(a)-(d)  that irradiances reach the peak  around the solar noon, and that  the largest sunshine  duration  and  irradiances  occur   in July. Also, values of diffuse irradiance  in comparison to direct  are smallest  in July, indicating  that  weaker  attenuation  is present  in July,  due to mostly clear sky conditions.  From average hourly irradiances  for each  day in a  given month, for  the period 2005-2020, we estimate average daily irradiations by months, i.e.  average energy which falls on    one square  meter during  a  typical day in a  given month.  Geographical coordinates  of   all five considered locations  for which calculations are done,  are given in Table 1. For each  town, in  Figure 4(a)-(d),  we present average daily irradiations on  a horizontal plane   in  January, April, July and October, i.e. for a month in each season. Daily average value of  GHI  is  the lowest for  January in   Kalinovik  (1.29 kWh/m2 ), whereas  the highest value is for  July  in    Bileća  (7.13 kWh/m2 ). Furthermore, as can be seen in Figure 4,  Bileća, Ljubinje and Berkovići  have  higher  and similar values  of corresponding daily  irradiations, whereas   Kalinovik and Rudo  have  lower,  mutually similar irradiations.

2.2  Average  monthly  irradiation,  diffuse to global  ratio and  temperature

PVGIS  provides   the data  of    GHI  for  each month over the period 2005-2020. Also, data of global irradiation  on a plane tilted at optimum angle (GOI) are available.  For a fixed inclined  plane,  global

| Town | Bileća | Ljubinje | Berkovići | Kalinovik | Rudo |
| --- | --- | --- | --- | --- | --- |
| Latitude | 42.874° | 42.952° | 43.117° | 43.504° | 43.618° |
| Longitude | 18.429° | 18.090° | 18.067° | 18.447° | 19.366° |
| Altitude | 473m | 408m | 870m | 1069m | 399m |
| Tilt angle | 36° | 34° | 36° | 33° | 34° |

Table 1.  Geographical coordinates (north lattitude and east longitude), altitude and  optimum tilt angle for  considered locations.

Figure 4.  Average daily global (GHI), beam  (BHI) and diffuse (DHI)  irradiation on a  horizontal  plane  for January (a), April (b), July (c) and October (d).

irradiation  is maximal at  optimum angle, and this option  should be considered while planning installation of PV modules. Optimum tilt angle is  given in  Table 1 for each considered town.  Another quantity of interest

is direct normal irradiation (DNI).  DNI is  direct irradiation on a plane  always facing normally to the Sun rays, which can be accomplished by tracking systems.   In Figure 5,   which presents average  monthly GHI calculated for each month,  one can see that  the lowest value of GHI, below 50kWh/m2, is for December in Kalinovik, whereas the highest monthly  values are over 220kWh/m2  for  July in  Bileća, Ljubinje and Berkovići.  Average global irradiation at optimum angle by months  is  presented in Figure 6, together  with numerical  values given tabularly. GOI is  the most relevant parameter concerning  our  PV  output estimates, and we will refer to these values in the  next section.  One can see that, compared to horizontal plane,  optimum angle significantly  increases   global irradiation  in non-summer months,  almost up to   100%   in winter.

High values of DNI and higher than GHI in non-summer months, are displayed  in Figure 7. In Figure 8,  average monthly diffuse to global ratio is shown, where we see how the ratio decreases towards summer months, when direct  irradiation dominates.  The ratio is the largest  for  Kalinovik and  Rudo in all months.    It is especially  large in winter when it reaches  67%  for  December in Kalinovik, for example.

Figure 5.  Average monthly  GHI  (by months) for five considered towns.

Figure 6.  Average monthly GOI  (by months)  for five considered towns.

Finally, average monthly temperature for all five towns is presented in Figure 9. We see that among all considered towns, Kalinovik has the lowest  average temperature  for  each  month during the year. It  is below zero degrees Celsius in  December, January and February, which is expected since   Kalinovik  is settled at  mountainous  region at an  altitude over 1000 m (Table 1).  Rudo has similar average temperature to Kalinovik, but somewhat higher.  Ljubinje has the highest average temperature among all considered locations,  slightly higher than Bileća  and Berkovići, with milder winters.

As an overall review, in Figure 10,  we present average yearly GHI, GOI and DNI. We see that  Bileća is in  a slight advantage,  e.g.  GHI is  1508  kWh/m2 per year,  while GOI is 1767 kWh/m2.   For Kalinovik  GHI is  1320  kWh/m2, while GOI is 1487 kWh/m2.  Average yearly values of  GOI for each location are  also presented in Table 2.

Figure 7.  Average monthly (by months)  DNI for five considered towns.

Figure 8. Average monthly diffuse to global                                Figure 9. Average monthly temperature for all five          irradiation ratio for all five towns.                                                       towns

Figure 10.  Average annually  GHI, GOI and DNI  for five considered towns.

Figure 11.  Average monthly  energy production from  PV crystalline  silicon  modul of 5kWp nominal power,  installed at fixed, optimum angle.

- ESTIMATES OF PV OUTPUT

In this section we present estimates of electrical energy that could be generated by grid connected PV modules installed at rooftops of households and  buildings.  We assume that the solar module is  crystalline silicon (c-SI) with the nominal (peak) power of 5kWp, and that it is fixed at  optimum angle.  Effects of terrain shadowing are accounted for through  the built in option ‘calculated horizon’. Besides  automatically accounted losses due to cell’s efficiency,  we add up an  additional loss of recommended 14% for  the overall system loss (cables, inverters…). Results for average  PV  output   by  months  (average electrical  energy in kWh  generated by the module   in each   month)   are given in Figure 11. We see that the largest  PV outputs over most of  the months  are for Bileća and  Berkovići, then Ljubinje.  Among all months, the largest outputs  are  in July,  and  they  vary  from about  668 kWh for  Rudo to 754 kWh for Berkovići. The smallest  outputs for  all towns are in December,  and they vary from 233 kWh for Kalinovik to 373kWh for  Bileća. From tabular values in Figure 11, it follows that,  except for winter months, the amount of generated electrical energy may  meet  demands of four-membered family,  with extra production in July and August. The corresponding values of energy that is received by  one square meter of the module are given in Figure 6, where  Global irradiation at optimum angle is shown graphically and tabularly. One should bear in mind that the overall surface  of  5kWp   module  is about  30 square meters, so  that  the total  energy  it    receives (incident energy) is 30 times larger than GOI.  From values given in  Figure 11 and the corresponding   values given in Figure 6 multiplied by surface,    it is obtained that 10-13% of solar energy   is converted into  electricity, depending on  town and month.   Although Bileća receives the largest amount of energy,  the largest  percentage i.e.  the best conversion is obtained   for Kalinovik and then Berkovići.   They are at higher altitudes where temperature, wind  and other  meteorological conditions might   give better working conditions for  PV modules.

As an overall view, in Table 2,  for each considered town,  we present average  daily and  monthly values throughout the whole year and average yearly, for the  period 2005-2020. The average values are given

for GOI – total energy received by  one square meter of the modules at optimum angle  and for  energy generated by the  whole module.

Table 2. Average daily, monthly and yearly values of  solar energy received by  one square meter of  5kWp module  set at  the fixed optimum slope and zero azimuth.  Corresponding outputs of the module  (the whole PV array ) are also given.

| Town | Geographical coordinates | Optimum tilt angle | Daily GOI kWh/m2 | Monthly GOI kWh/m2 | Yearly GOI kWh/m2 | Daily output kWh | Monthly output kWh | Yearly output kWh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bileća | 42.874 N 18.429 E | 36° | 4.84 | 147.23 | 1766.78 | 18.02 | 548.20 | 6578.38 |
| Ljubinje | 42.952 N 18.090 E | 34° | 4.66 | 141.64 | 1699.65 | 17.04 | 529.14 | 6349.68 |
| Berkovići | 43.117 N 18.067 E | 36° | 4.74 | 144.16 | 1729.91 | 18.03 | 548.35 | 6580.17 |
| Kalinovik | 43.504 N 18.447 E | 33° | 4.07 | 123.89 | 1486.73 | 15.96 | 485.35 | 5824.25 |
| Rudo | 43.618 N 19.366 E | 34° | 4.11 | 124.88 | 1498.52 | 15.32 | 466.13 | 5593.54 |

- SUMMARY AND CONCLUSIONS

In this paper we have  estimated and analyzed  solar energy potential of five towns, capitals of the same-named  municipalities, located   in the   southeastern  part of the   Republic of Srpska.  Average  daily GHI, BHI and DHI are  presented for a month in each season  in Figure 4 (a)-(d), whereas average  daily  value of GOI over a whole year is given in  Table 2. Average  monthly GHI, GOI, and DNI for each month  are  given in  Figures 5-7,  whereas average monthly  GOI over a whole year  is given in Table 2. Average  yearly  GOI  is given in  Table 2.  From the values of  these parameters, it is evident that  Bileća, Ljubinje and Berkovići,  with  average annual GOI  of  about 1700 kWh/m2,  are municipalities with exquisite conditions for solar energy harnessing. Regarding  solar  potential they are almost comparable with  the southern coastal part of Croatia, and   more favorable   than  Serbia in average [19].  Kalinovik and Rudo  have average annual GOI  of  about  1500 kWh/m2, which is still significant,  and  is  comparable with  Pannonian part of Serbia and Croatia [20], for example.

We  have also estimated the amount of electrical energy that could  be produced by solar modules  with a  nominal power of  5kWp,  possibly installed on the rooftops at fixed optimum angle and oriented south.  Average monthly PV output  for  each month and each town is presented in Figure 11, whereas average daily and monthly values over a year and average annual values are presented in Table 2.  Berkovići and Bileća  have the largest annual outputs, about 6600kWh of electricity generated per year, which is  more than energy demand of an average household.  For all considered locations, the amount of  generated electricity  can cover the household’s needs in average.  In winter,  grid supplement  might be necessary,  whereas in summer, an excess energy could be exported to the utility grid.

One should  remark that  the values of irradiations  obtained and  presented in this paper are just the estimates made on the basis of satellite provided data for the given coordinates. Temporal resolution of  the  data are one hour, while spatial resolution is 5 km.  Besides, landscape  near the chosen locations can produce additional  shadowing effects,  which were not taken into account in calculations.  For these reasons, more precise  assessment would require  finer resolution and  all the estimates should be validated with the data produced by ground station measurements.

Also, we would like to mention that,   since  most rooftops are already inclined,  the panels could  be  mounted at  roof angle,  without additional tilt frames.   Then, estimates can be made for any given angle. This would result in smaller outputs than presented here, depending on the  deviation of roof angle from optimum tilt angle. In most cases, for  small power plants,  it might not be   financially profitable to  make adjustments  for optimum angle.

Finally,  as we found that the   considered  area  is   very favorable for solar harnessing, we would like to mention that   this study can easily be extended  on estimates of PV output for large  solar power plants  with tracking systems, which would be the subject of our  future work.

- ACKNOWLEDGEMENT

This research is   financially supported   by the Ministry of Scientific and Technological Development and  Higher Education of the Republic of Srpska  through  the  project  No.  19.032/961-26/23 approved on 29.12.2023.

- REFERENCES

[1] T. M.  Letcher, V. M. Fthenakis (eds.) A comprehensive Guide to Solar  Energy Systems, Academic Press 	Elsevier, London, 2018.

[2]  M. E. Mackay, Solar energy- An Introduction, Oxford University Press, Oxford UK, 2015.

[3]  R. Foster, M. Ghassemi, A. Cota, Solar Energy: Renewable Energy and Environment, CRC Press, Boca

Raton, 2010.  											         [4] IRENA, World Energy Transitions Outlook 2023, https://www.irena.org/Publications/2023/Jun/World-Energy-Transitions-Outlook-2023

[5] F. Sher, N. Smječanin, H. Hrnjić, E. Bakunić, J. Sulejmanović,  Prospects of renewable energy potentials and development in Bosnia and Herzegovina – A review, Renew. Sustain. Energy Rev., 189  (2024) 113929.

[6]    Law on renewable energy sources,  Official Gazette of the Republic of Srpska, no.16/22, Banja Luka, 2022.

https://www.derk.ba/DocumentsPDFs/Zakon-o-obnovljivim-izvorima-energije-RS-16-2022-h.pdf

[7] EU Science Hub, https://joint-research-centre.ec.europa.eu/photovoltaic-geographical-information-system-pvgis_en

[8] M. Šúri, T. A. Huld, E. D. Dunlop, PV-GIS: a web-based solar radiation database for the calculation of  PV potential in Europe, Int. J. Sustain. Energy, 24 (2005) 55-67.

[9] M. Šúri,T. A. Huld, T. Cebecauer, E. D. Dunlop, Geographic Aspects of Photovoltaics in Europe:Contribution of the PVGIS Website, IEE Journal of selected topics in applied Earth observations and remote sensing, 1 (2008) 1939-1404.

[10] I. Radonjić Mitić, T. Pavlović, D. Mirjanić, D. Divnić, L. Pantić, Reviewing the photovoltaic potential of Bijeljina in the Republic of Srpska,  Facta Universitatis, Series: Electronics and Energetics 36 (2023), 465-483.

[11] D. LJ. Mirjanić, T. M.  Pavlović, I. S. Radonjić, L. S. Pantić, G. I. Sazhko, Solar radiation atlas in Banja Luka in the Republic of Srpska, Contemporary Materials, XII˗1, (2021), 29-49.

[12] D. Lj. Mirjanić, T. M.  Pavlović, I. S. Radonjić, L. S. Pantić,  A. V. Marić, G. I. Sazhko, Solar radiation atlas in Trebinje in the Republic of Srpska, Proceedings from International Scientific Conference “UNITECH 2020” – Gabrovo, 20-21 November 2020, pp. I-33 – I-38

[13] D. D. Milosavljević, T. M. Pavlović, D. Lj. Mirjanić, D. Divnić, Photovoltaic solar plants in the Republic of Srpska - current state and perspectives, Renew. Sustain. Energy Rev., 62  (2016) 546-560.

[14] T. M. Pavlović, D. D. Milosavljević,   D. Mirjanić, L. S. Pantić, I. S.  Radonjić,   D. Piršl, Assessment and perspectives of PV solar power engineering in the Republic of Srpska (Bosnia and Herzegovina), Renew. Sustain. Energy Rev., 18  (2013) 119-133.

[15] T. M.  Pavlović, , D. D. Milosavljević, D. Lj. Mirjanić, I. S. Radonjić, L. S. Pantić, D. Piršl,   Analyses of PV systems of 1 kW electricity generation in Bosnia and Herzegovina, Contemporary Materials (Renewable energy sources) II-2 (2011) 123–138.

[16] D. Mirjanić, S. Maksimović, D. Divnić,  T. Pavlović L. Pantić, D. Milosavljević, Investigation of energy efficiency of polycrystalline silicon solar modules in relation to their geographical orientation and tilt angle, Contemporary Materials (Renewable energy sources) VI-2 (2015) 87–94.

[17] UNFCC Bosnia and  Herzegovina, https://www.unfccc.ba/en/analize-en/klimatskekarakteristikebih-en

[18] SOLARGIS, Global Solar Atlas, https://globalsolaratlas.info/map

[19] T. Pavlović, D. Milosavljević,  I. Radonjić, L. Pantić,  A.  Radivojević, M. Pavlović, Possibility of electricity

generation using PV solar plants in Serbia, Renew. Sustain. Energy Rev., 20  (2013) 201-218.

[20] B. N. Smaragdakis, D. Šljivac, V. Katić, T. Stajić, Z. Čepić, D. Topić, M. Vukobratović, Solar energy potential in

Pannonian part of Serbia and Croatia, International Journal of Electrical and Computer Engineering Systems, 3-1 (2012),

31-39.

Daily average over a  whole year could  be calculated  as a  weighted average of daily averages for each month, and these values would vary in the same  range as those given in the map  in Figure 2.

For a module with peak efficiency eff=0.15, the surface  of the module would be A=5kWp1kWpm2*0.15≈33m2.