Међународни научни скуп Савремени материјали –  Бања Лука.

International Scientific Conference Contemporary Materials – Banja Luka 

ИСТРАЖИВАЊЕ ЕНЕРГЕТСКЕ ЕФИКАСНОСТИ ТОПЛОТНИХ ПУМПА ЗА ГРЕЈАЊЕ


Lulić, Valentina.1, Golubović, Dušan.1, Savković Borislav.2, Kovač Pavel.2, Ješić, Dušan.3, Dudić, Branislav.4,5
1Faculty of Mechanical Engineering, University of East Sarajevo, 71123 East Sarajevo, Vuka Karadžića 30, Bosnia and Hercegovina, e-mail: valentinalulic313@gmail.com  dusan.golubovic54@gmail.com 
2University of Novi Sad, Faculty of Technical Science, Trg D. Obradovića 7, 21000 Novi Sad, Serbia, e-mail: pkovac@uns.ac.rs, savkovic@uns.ac.rs 
3International Technology Management Academy, Trg D.Obradovića 7, 21000 Novi Sad, Serbia, e-mail: dusanjesic@hotmail.com

4Comenius University, Faculty of Management, Bratislava, Slovakia, e-mail: branislav.dudic@fm.uniba.sk
5University Business Academy, Faculty of Economics and Engineering Management, Serbia.
Сажетак: Овај чланак представља компоненте топлотних пумпи, циклусе хлађења, пример примене топлотне пумпе и њену економичност. Акценат у чланку је да се прикаже енергетска ефикасност топлотне пумпе у односу на одговарајућа котловска постројења. Урађена је економска анализа појединачних решења према економским критеријумима на основу којих се могу бирати предложене варијанте. Те анализе претпостављају техничке концепте који директно одређују степен профитабилности одабраног система. Овакве анализе обично никада нису рађене за класичне енергетске системе, па је, пошто смо били приморани да их радимо без припреме, прелазак на нове технолошке концепте добијања топлотне енергије био и још увек је успорен.
Кључне ријечи: топлотне пумпе, енергија, ефикасност, економичност, котларнице.
RESEARCH OF ENERGY EFFICIENCY OF HEAT PUMPS FOR HEATING
Abstract: This article presents the components of heat pumps, cooling cycles, an example of heat pump application and its economy. The emphasis in the article is to show the energy efficiency of the heat pump in relation to the corresponding boiler plants. An economic analysis of individual solutions was performed according to economic criteria on the basis of which the proposed variants can be selected. Those analyzes assume technical concepts that directly determine the degree of profitability of the selected system. Such analyzes have usually never been done for classic energy systems, so since we were forced to do them without preparation, the transition to new technological concepts of obtaining heat energy was and still is slowed down.
Keywords: heat pumps, energy, efficiency, economy, boiler plants. 

