POSSIBILITY OF PRODUCING PHOTOVOLTAIC ELECTRICITY ON BALCONY FENCE IN PIROT, SERBIA

Ivana Radonjić1, Milutin Petronijević2, Leonid Stoimenov2, Dragoljub Mirjanić3, Lana Pantić1, Darko Divnić3

1 University of Niš, Faculty of Sciences and Mathematics, Department of Physics, Višegradska 33, 18000 Niš, Serbia

2 University of Niš, Faculty of Electronic Engineering, A. Medvedeva 14, 18000 Niš, Serbia

3 Academy of Sciences and Arts of the Republic of Srpska, Bana Lazarevića 1, 78000 Banja Luka, Republic of Srpska, Bosnia and Herzegovina

Abstract: In urban environments, PV panels are usually affixed on available buildings, but they can also be included in the initial architectural design of the constructions. PV panels mounting on/as balcony fences become frequent solution for creating PV systems in buildings. The idea of ​​decentralized production and storage of electricity very close to the point of use represents an additional benefit. In this paper, the possibility of using PV panels in urban conditions, within the home installations of individual residential consumers with the placing of panels on the balcony fence, is analyzed. A detailed simulation of the complete PV system in the PVSYST software was used for the analysis of different mounting options for PV panels on the balcony fence. For an individual consumer from the "Household type 3" category with an annual consumption of about 6000 kWh, the installation of PV panels with a total power of 1650 Wp was analyzed. Different variants of installation of PV panels were simulated for the case of two different tilt angles (0o (vertical position) and 60o) and three different azimuthal orientations (south, east and west). Self-consumption is modeled through typical daily profiles, taking into account seasonal variation for a given type of consumer. The obtained results show that in the investigated PV system, photovoltaic energy can cover from 18% to 35% of considered household’s self-consumption, depending on the tilt angle of the PV panels on the balcony fence and their azimuthal orientation.

Keywords: PV panel, balcony fence, electricity production, electricity consumption, household

МОГУЋНОСТ ПРОИЗВОДЊЕ ФОТОНАПОНСКЕ ЕЛЕКТРИЧНЕ ЕНЕРГИЈЕ НА ОГРАДИ БАЛКОНА У ПИРОТУ, СРБИЈА

Сажетак: У урбаном окружењу фотонапонски (PV) панели се обично постављају на расположиве зграде, али се могу укључити и у почетни архитектонски дизајн грађевина. Монтажа PV панела на/као балконске ограде постаје често решење за креирање PV система у зградама. Идеја децентрализоване производње и складиштења електричне енергије врло близу места коришћења представља додатни бенефит. У овом раду је анализирана могућност коришћења PV панела у градским условима, унутар кућних инсталација индивидуалног стамбеног потрошача са постављањем панела на огради балкона. За анализу различитих варијанти монтаже PV панела на огради балкона коришћена је детаљна симулација комплетног PV система у софтверу PVSYST. За индивидуалног потрошача из категорије „Домаћинство тип 3“ са годишњом потрошњом од око 6000 kWh, анализирана је уградња PV панела укупне снаге 1650 Wp. Симулиране су различите варијанте уградње PV панела и то за случај два различита угла нагиба (0o (вертикални положај) и 60o) и три различите азимутне оријентације (југ, исток и запад). Сопствена потрошња је моделована преко типичних дневних профила уз уважавање сезоналне варијације за дати тип потрошача. Добијени резултати показују да у испитиваном PV систему фотонапонска енергија може да покрије од 18% до 35% сопствене потрошње посматраног домаћинства, а у зависности од угла нагиба PV панела на огради балкона и њихове азимутне оријентације.

Кључне ријечи: PV панел, ограда балкона, производња електричне енергије, потрошња електричне енергије, домаћинство