Еволуционе Стратегије за Процену Електрофизиолошких Параметара

Evolutionary Strategies for Electrophysiological Parameters Estimation

Богдан Милићевић1,2, Ненад Филиповић2,3

1Институт за информационе технологије, Универзитет у Крагујевацу, Крагујевац, bogdan.milicevic@uni.kg.ac.rs

2Истраживачко развојни центар за биоинжењеринг, Крагујевац

3Факултет инжењерских наука, Универзитет у Крагујевацу, Крагујевац, fica@kg.ac.rs

Сажетак Прецизна процена електрофизиолошких параметара од суштинског је значаја за разумевање и моделовање динамичког понашања срчаног ткива током целог срчаног циклуса, који обухвата фазе деполаризације, реполаризације и рефрактерног периода. Ове фазе представљају основу за настанак и ширење акционих потенцијала који координишу ритмичке контракције срца. У овом раду представљамо оквир за процену параметара који се ослања на еволутивне стратегије, конкретно на метод Диференцијалне еволуције (DE), као робустан алат за идентификацију скривених параметара у нелинеарним моделима срчане електрофизиологије. DE, као глобални оптимизациони алгоритам заснован на популацији, посебно је погодан за проблеме са високим степеном нелинеарности и димензионалности, што је карактеристично за биолошке системе.

Abstract: Accurate estimation of electrophysiological parameters is essential for understanding and modeling the dynamic behavior of cardiac tissue throughout the entire cardiac cycle, which includes the phases of depolarization, repolarization, and the refractory period. These phases form the basis for the generation and propagation of action potentials that coordinate the rhythmic contractions of the heart. In this paper, we present a parameter estimation framework based on evolutionary strategies, specifically the Differential Evolution (DE) method, as a robust tool for identifying hidden parameters in nonlinear cardiac electrophysiology models. DE, as a population-based global optimization algorithm, is particularly well-suited for addressing problems characterized by high nonlinearity and dimensionality, which are inherent to biological systems.

Kључне ријечи: лева комора, еволуциони алгоритми, оптимизација, електрофизиологија, акциони потенцијали

Keywords: left ventricle, evolutionary computation, optimization, electrophysiology, action potentials

Acknowledgement: This study has received funding from the European Union’s  Horizon Europe research and innovation programme under grant agreement No 101080905 (STRATIFYHF). Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or granting authority – European Health and digital executive agency (HADEA). Neither the European Union nor the granting authority can be held responsible for them. This research was also supported by the Ministry of Science, Technological Development and Innovation of the Republic of Serbia, contract number [451-03-136/2025-03/200378 (Institute of Information Technologies, University of Kragujevac) and 451-03-136/2025-03/200107 (Faculty of Engineering, University of Kragujevac)].