Процена могућности употребе гласовних карактеристика као неинвазивних биомаркера у дијагностици срчане слабости

Evaluating the Potential of Vocal Features as Non-Invasive Biomarkers for Heart Failure

Тијана Героски1,2, Лазар Дашић2,3, Анђела Благојевић1,2, Огњен Павић2,3, Ненад Филиповић1,2

1Факултет инжењерских наука, Универзитет у Крагујевацу, Крагујевац

2Истраживачко развојни центар за биоинжењеринг, Крагујевац

3Институт за информационе технологије, Универзитет у Крагујевацу, Крагујевац

E-mails: tijanas@kg.ac.rs, lazar.dasic@kg.ac.rs, andjela.blagojevic@kg.ac.rs, opavic@kg.ac.rs, fica@kg.ac.rs

Сажетак Овај рад истражује потенцијал гласовних карактеристика као неинвазивних биомаркера за детекцију срчане слабости. Промене у гласу могу бити последица поремећаја респираторног и кардиоваскуларног система, што глас чини обећавајућим извором клиничких информација. У оквиру истраживања, дефинисан је стандардан протокол снимања који обухвата пет задатака: (1) читање текста нормалним темпом, (2) слободан говор у трајању од најмање 30 секунди, (3) бројање од 1 до 30, (4) бројање уназад од 30 до 1, и (5) максимално фоновање гласа „ааа“. Из снимака се издвајају различите акустичке карактеристике као што су: дужина фоновања, број и трајање пауза, висина и гласноћа гласа, мел-фреквенцијски кепстрални коефицијенти (MFCC), јитер, шиммер и цепстрална истакнутост (CPP). У току су клиничке студије са циљем процене ефикасности ове методологије и њене примене у реалним условима као алата за рану, неинвазивну дијагностику и праћење болести. Овај приступ има потенцијал да олакша самопраћење пацијената и смањи трошкове здравственог система.

Abstract: This paper explores the potential of vocal features as non-invasive biomarkers for heart failure detection. Changes in voice may result from disruptions in the respiratory and cardiovascular systems, making voice a promising clinical signal. A standardized recording protocol was defined, consisting of five tasks: (1) reading a text at a normal speaking pace, (2) free speech for at least 30 seconds, (3) counting from 1 to 30, (4) counting down from 30 to 1, and (5) a maximum phonation task (“aaa” for as long as possible). From these recordings, various acoustic features are extracted, including phonation duration, number and length of pauses, pitch, loudness, mel-frequency cepstral coefficients (MFCCs), jitter, shimmer, and cepstral peak prominence (CPP). Clinical studies are currently underway to assess the effectiveness of this methodology and its applicability in real-world settings as a tool for early, non-invasive diagnosis and monitoring of heart failure. This approach could empower patient self-management and help reduce healthcare costs.

Kључне речи: срчана слабост, гласовни биомаркери, анализа гласа, неинвазивна дијагностика

Keywords: heart failure, vocal biomarkers, voice analysis, non-invasive diagnosis

Acknowledgement: This study has received funding from the European Union’s Horizon Europe research and innovation programme under grant agreement No 101080905 (STRATIFYHF). Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or granting authority – European Health and digital executive agency (HADEA). Neither the European Union nor the granting authority can be held responsible for them. This research was also supported by the Ministry of Science, Technological Development and Innovation of the Republic of Serbia, contract numbers [451-03-136/2025-03/200107 (Faculty of Engineering, University of Kragujevac) and 451-03-136/2025-03/200378 (Institute of Information Technologies, University of Kragujevac)].