# ECONOMIC AND ENVIRONMENTAL ASPECTS OF WASTE MANAGEMENT –"METERIS" LANDFILL CASE STUDY

Jelena Marković1, Goran Marković2, Ivan Stevović3

1 Academy of Technical Educational Vocational Studies Department Vranje, Filipa Filipovića 17500 Vranje, Serbia, jelena.markovic@akademijanis.edu.rs

2 Public Institution National University of Vranje, narodniuniverzitet@gmail.com

3 Innovation Center of the Faculty of Mechanical Engineering, University of Belgrade, Serbia

https://orcid.org/0000-0003-4729-9241, istevovic@mas.bg.ac.rs

# Abstract: Industrial waste management has become one of the major challenges of contemporary environmental protection systems due to its significant impacts on soil, water, air quality, and public health. This study examines the economic and environmental dimensions of waste management through a case study of the Meteris landfill located in Vranje, Serbia. The research is based on the analysis of operational documentation, statistical records of disposed waste covering the period from 2018 to 2023, and the relevant legislative and institutional framework governing waste management. Particular attention is given to identifying the principal challenges associated with industrial waste management, including limited waste separation and treatment capacities, insufficient implementation of circular economy principles, and inadequate institutional coordination among stakeholders. The findings indicate that although certain improvements have been achieved in landfill operation and waste handling practices, landfilling remains the dominant disposal method, while recycling and material recovery rates remain relatively low. The study emphasizes the necessity of modernizing waste management systems through investments in recycling infrastructure, advanced waste treatment technologies, continuous environmental monitoring, and stronger enforcement of environmental regulations. The proposed measures could significantly reduce environmental risks, improve resource efficiency, and support the transition toward sustainable waste management practices consistent with the principles of the circular economy.

Keywords: industrial waste materials; waste management; Meteris landfill; circular economy; environmental protection; sustainable development.

# 1. INTRODUCTION

Industrialization, urban expansion, and increasing consumption have led to a continuous growth in waste generation worldwide, making waste management one of the most significant environmental, economic, and social challenges of the twenty-first century. Inadequate waste management practices contribute to soil degradation, groundwater contamination, greenhouse gas emissions, and biodiversity loss, while simultaneously posing considerable risks to human health. Consequently, sustainable waste management has become an essential component of environmental protection policies and an important prerequisite for achieving the Sustainable Development Goals (SDGs), particularly those related to responsible consumption and production, climate action, and sustainable cities [1].

Industrial waste represents a particularly complex category of waste because of its heterogeneous composition and the potential presence of hazardous substances [2]. Depending on the industrial sector, waste streams may contain chemicals, heavy metals, plastics, construction materials, electronic components, and biodegradable fractions that require different treatment technologies and disposal methods. If these materials are improperly managed, they may cause long-term environmental degradation and substantial economic losses associated with remediation and public health protection.

Modern waste management systems increasingly rely on the principles of the circular economy, which prioritize waste prevention, reuse, recycling, and material recovery over final disposal [3]. This approach transforms waste from an environmental burden into a valuable secondary resource, thereby reducing the demand for virgin raw materials and improving resource efficiency. In addition to environmental benefits, circular economy strategies generate economic value through lower disposal costs, increased recycling revenues, and improved industrial competitiveness.

The Republic of Serbia has made significant progress in harmonizing its environmental legislation with European Union directives [4]. Nevertheless, numerous challenges remain, particularly regarding infrastructure development, selective waste collection, recycling capacities, and effective enforcement of environmental regulations. These challenges are especially evident at regional landfills, where mixed municipal and industrial waste streams continue to dominate. The regional landfill "Meteris", located in the village of Suvi Dol near the City of Vranje, represents one of the most important waste disposal facilities in southeastern Serbia [5][6][7]. Besides municipal waste, the landfill receives waste generated by commercial and industrial activities within the region, making it an appropriate case study for evaluating current waste management practices and identifying opportunities for improvement.

The primary objective of this study is to evaluate the economic and environmental aspects of industrial waste management at the Meteris landfill. More specifically, the study aims to:

- analyze the characteristics and quantities of disposed waste during the period 2018–2023;

- examine existing waste treatment and disposal practices;

- identify the main environmental and economic challenges associated with landfill operation;

- assess the current level of compliance with sustainable waste management principles; and

- propose recommendations for improving waste management efficiency in accordance with circular economy principles and environmental sustainability.

The scientific contribution of this study lies in integrating environmental, economic, and operational perspectives into a comprehensive case study of a regional landfill in Serbia. The findings provide practical recommendations for improving landfill management while contributing to the broader discussion on sustainable industrial waste management in transition economies.

# 2. METHODOLOGY

## 2.1 Research Design

This study applies a qualitative–quantitative case study approach to evaluate the economic and environmental aspects of industrial waste management at the Meteris landfill in Vranje, Serbia. The case study methodology was selected because it enables a comprehensive assessment of waste management practices within a real operational environment while simultaneously considering technical, environmental, economic, and institutional factors.

The research combines descriptive, comparative, and analytical methods to examine existing waste management procedures and their implications for environmental sustainability. Quantitative analysis was used to assess annual waste generation trends and waste composition between 2018 and 2023, whereas qualitative analysis focused on evaluating operational procedures, environmental protection measures, and compliance with applicable regulations. The adopted methodological framework allows the integration of multiple sources of evidence, thereby increasing the reliability of the research findings and providing a holistic understanding of landfill management practices.

## 2.2 Study Area

The research was conducted using the Meteris regional landfill, located in the settlement of Suvi Dol near the City of Vranje in southern Serbia, as the case study, Figure 1.

Figure 1. The research area in the Suvi Dol

The landfill serves as the primary municipal waste disposal facility for the City of Vranje and surrounding municipalities and receives municipal, commercial, and selected categories of industrial waste generated within the region. Although municipal waste constitutes the dominant waste stream, the landfill also accepts industrial waste from manufacturing companies operating in the Vranje area. This combination of waste streams makes the landfill an appropriate case for evaluating interactions between municipal and industrial waste management practices.

The landfill performs several operational activities, including waste reception, weighing, compaction, temporary storage, partial separation of recyclable materials, and final disposal. These activities were considered in assessing both operational efficiency and environmental performance.

## 2.3 Data Collection

The study is based on secondary data analysis obtained from several complementary sources, dominantly related to the landfill Meteris, belonging to the municipality Vranje.

The primary dataset consists of annual operational records documenting waste quantities disposed of at the Meteris landfill during the period 2018–2023. These records include information on:

- total annual quantities of disposed waste (t);

- waste volume (m³);

- distribution according to waste categories;

- industrial waste delivered by selected companies;

- recyclable waste fractions;

- operational characteristics of landfill activities.

To complement these quantitative data, the study also examined publicly available documentation related to waste management practices, landfill operation, and environmental protection measures.

Furthermore, the research incorporates an analysis of the applicable legal and institutional framework governing waste management in the Republic of Serbia. Relevant national legislation regulating waste management, environmental protection, and landfill operation was reviewed to evaluate the degree of compliance between existing operational practices and regulatory requirements. The legislative review also considers Serbia's ongoing harmonization with the European Union environmental acquis, particularly with respect to sustainable waste management and circular economy principles.

A review of contemporary scientific literature was additionally performed to provide the theoretical background necessary for interpreting the obtained results and comparing them with findings reported in previous international studies.

## 2.4 Data Analysis

The collected data were analysed using a combination of descriptive and comparative statistical methods.

Descriptive statistical analysis was employed to summarize annual waste quantities, identify dominant waste categories, and evaluate temporal changes during the analysed period. Comparative analysis enabled the identification of trends in industrial waste generation and landfill utilization between individual years.

Trend analysis was conducted to determine whether the quantity of disposed waste exhibited increasing or decreasing tendencies over time. Particular attention was paid to the relationship between municipal and industrial waste streams, as well as changes in recyclable material recovery.

The qualitative component of the analysis focused on evaluating operational waste treatment procedures, including waste reception, separation, compaction, temporary storage, and final disposal. These procedures were assessed from environmental and economic perspectives by considering their contribution to pollution prevention, landfill capacity optimization, and resource recovery.

Finally, the obtained findings were interpreted within the framework of sustainable waste management and circular economy concepts. This approach enabled an assessment of current management practices relative to internationally recognized waste hierarchy principles, emphasizing waste prevention, reuse, recycling, recovery, and environmentally sound disposal.

## 2.5 Reliability and Research Limitations

The reliability of the research is primarily based on the use of official operational records collected over a seven-year period, allowing the identification of long-term waste management trends rather than isolated annual observations. The integration of quantitative operational data with qualitative analysis and legislative review further enhances the credibility and consistency of the study. Nevertheless, several limitations should be acknowledged.

First, the research relies predominantly on secondary data obtained from landfill operational records, without conducting direct laboratory analyses of soil, groundwater, leachate, or landfill gas emissions. Consequently, the study evaluates environmental impacts indirectly through documented operational practices and available monitoring information.

Second, the analysis is limited to a single regional landfill. Although the Meteris landfill represents an important example of waste management practice in southern Serbia, the findings cannot be generalized to all Serbian landfills without considering regional differences in infrastructure, operational capacity, and waste composition.

Despite these limitations, the selected methodological framework provides a reliable basis for assessing the current state of industrial waste management and identifying opportunities for improving environmental performance and economic efficiency.

## 2.6 Ethical Considerations

This study was conducted in accordance with the principles of scientific integrity and research ethics. The analysis is based exclusively on aggregated operational and publicly available data, without involving human participants or processing personal or confidential information. Consequently, ethical approval was not required under the applicable institutional guidelines. All data were used solely for scientific purposes, and the findings are presented objectively to support evidence-based recommendations for improving sustainable waste management practices.

# 3. RESULTS

## 3.1. Waste Management Aspects

The environmental aspects of industrial waste management include all potential impacts that waste may have on the environment. Improper storage and disposal of waste can result in the contamination of soil, surface water, and groundwater. Emissions of harmful gases from industrial waste negatively affect air quality and contribute to climate change. In addition, waste may pose a serious threat to flora and fauna by disrupting biodiversity. Therefore, it is essential to conduct comprehensive environmental impact assessments and implement appropriate environmental protection measures.

Industrial waste management also has significant economic implications. The costs associated with waste collection, treatment, transportation, and final disposal represent a considerable expenditure for industrial facilities. In many cases, inadequate waste management results in additional financial burdens through environmental penalties, remediation costs, and reputational damage. Consequently, efficient waste management is essential not only from an environmental perspective but also for ensuring long-term economic sustainability.

One of the key economic objectives is reducing costs through improved recycling and material recovery processes. Recycling and the reuse of secondary raw materials contribute to resource conservation while decreasing the demand for virgin raw materials, thereby lowering operational costs over the long term. In modern waste management systems, recycling is increasingly recognized as an investment rather than an expense.

Furthermore, environmental taxes, incentives, and government subsidies represent important policy instruments for promoting responsible waste management practices. Industrial companies that invest in environmentally friendly technologies and sustainable production processes may benefit from financial incentives or tax relief. In this way, economic policy encourages market participants to adopt practices that support sustainable development.

The "Meteris" landfill primarily receives municipal solid waste generated by households, public institutions, and small businesses within the City of Vranje and its surrounding settlements. Most of the waste consists of mixed municipal waste that is not separated at the source, making recycling considerably more difficult. Organic waste, paper, plastics, textiles, and glass are commonly disposed of together without prior sorting.

In addition to municipal waste, the landfill also receives construction and demolition waste, including rubble and excavated soil. Although this category of waste poses fewer environmental risks than biodegradable or hazardous waste, it occupies substantial landfill capacity.

A major environmental concern is the disposal of non-biodegradable plastic materials, particularly plastic packaging and household plastic bags. Wind frequently disperses lightweight plastic waste into the surrounding environment, contaminating nearby land and watercourses. Since plastics degrade extremely slowly, they gradually fragment into microplastics, which pose long-term risks to ecosystems. The presence of potentially hazardous household waste, including batteries, electronic components, and chemical residues, further increases environmental risks due to the possible release of heavy metals and other toxic substances into soil and groundwater.

## 3.2. Waste Treatment Prior to Disposal

At the "Meteris" landfill in Vranje, several waste treatment procedures are implemented before final disposal. The treatment process primarily involves waste separation, during which recyclable materials are extracted for recovery and potential reuse. This procedure reduces the quantity of waste requiring final disposal and contributes to more efficient resource utilization.

The remaining waste undergoes additional physical and mechanical treatment, mainly through shredding and compaction. These operations improve the utilization of landfill space while reducing the formation of landfill gases generated during the decomposition of biodegradable waste. Although a comprehensive biological or chemical waste treatment system has not yet been fully implemented at the landfill, further modernization and technological improvements are planned. Waste is periodically covered with soil to minimize unpleasant odors, reduce the spread of litter by wind, limit pest activity, and improve overall environmental conditions at the landfill site. The primary objective of pre-disposal treatment is to minimize the environmental impacts of waste while increasing the efficiency of landfill operations. Consequently, the landfill functions not only as a disposal facility but also as an important component of the regional waste management system and the transition toward a circular economy. Although current technical capacities remain limited, existing treatment practices demonstrate continuous progress and provide a solid basis for future technological development.

## 3.3. Industrial Waste Disposal at the "Meteris" Landfill during the Period 2018–2023

The analysis of waste disposal data for the period 2018–2023 indicates that the quantity of waste deposited at the "Meteris" landfill has generally increased over time, although a gradual decline was observed after 2020. Efficient daily waste compaction has significantly extended the operational lifespan of the landfill by improving the utilization of available disposal space.

In 2018, the total quantity of disposed waste amounted to 17,044.6 t, with biodegradable waste representing the largest fraction (8,064.0 t). Garden waste also accounted for a considerable proportion (1,400.8 t), while disposable diapers represented 444.1 t. Plastic packaging (1,113.3 t) and cardboard (1,472.8 t) were among the most significant recyclable fractions, indicating substantial potential for material recovery. The total waste volume reached 48,130 m³.

The total quantity of disposed waste increased to 19,924.8 t in 2019, with a total volume of 61,496 m³. Biodegradable waste remained the dominant fraction, exceeding 9,400 t, while the quantities of plastic packaging, garden waste, disposable diapers, and fine particles also increased. Industrial waste data identified Geox, Alfaplam, and Yumco as the largest industrial waste generators, reflecting their high production capacities.

The highest amount of waste was recorded in 2020, when total disposal reached 24,740.89 t. Municipal waste accounted for 21,837.7 t, while commercial and industrial waste amounted to 1,299.4 t and 1,604.1 t, respectively. These results demonstrate that municipal waste remains the dominant waste stream, although commercial and industrial waste continue to represent a significant burden on landfill capacity.

From 2021 onward, a gradual decline in disposed waste quantities was observed. Total waste decreased to 23,331.49 t in 2021, 21,439 t in 2022, and 19,916.49 t in 2023. Despite this reduction, municipal waste consistently represented the largest proportion of disposed waste, while industrial waste remained comparatively low.

The analysis of recyclable materials delivered during 2022 revealed that PET packaging and cardboard constituted the majority of recovered materials, whereas no deliveries of metal or glass were recorded during the analysed period. These findings suggest that selective collection systems for certain recyclable fractions remain insufficiently developed and require further improvement.

Overall, the results indicate that landfill disposal continues to be the predominant waste management practice at the "Meteris" landfill. Although waste separation, recycling, and pre-disposal treatment activities are implemented to a certain extent, substantial opportunities remain for increasing recycling rates, expanding biological waste treatment, improving source separation, and reducing the quantity of waste requiring final disposal. These measures would contribute to both environmental protection and the long-term economic sustainability of the regional waste management system.

### 3.4. Summary of Results

The analysis of waste management practices at the "Meteris" landfill indicates that municipal waste remains the dominant waste stream throughout the observed period, while industrial and commercial waste account for a considerably smaller share of the total disposed quantities. Nevertheless, the continuous presence of industrial waste highlights the importance of maintaining an organized collection, treatment, and disposal system in order to minimize potential environmental impacts. The results presented in Table 1 show that the total quantity of disposed waste generally increased between 2018 and 2020, reaching its highest value in 2020. From 2021 onward, a gradual decline in total waste quantities was recorded, suggesting improvements in waste management practices, changes in waste generation patterns, or increased implementation of waste reduction measures. Despite this decrease, the overall amount of waste disposed at the landfill remains  substantial, confirming the continuing pressure on landfill capacity.

Table 1. Summary of annual waste quantities and landfill utilization at the Meteris landfill (2018–2023)

| Year | Total waste (t) | Waste volume (m³) | Dominant waste category | Main observation |
| --- | --- | --- | --- | --- |
| 2018 | 17,044.6 | 48,130 | Biodegradable waste | High biodegradable fraction, considerable composting potential |
| 2019 | 19,924.8 | 61,496 | Biodegradable waste | Increase in plastic packaging and municipal waste |
| 2020 | 24,740.9 | 86,790 | Municipal waste | Highest annual waste quantity recorded |
| 2021 | 23,331.5 | 9,216.3 | Municipal waste | Reduced waste volume, improved compaction efficiency |
| 2022 | 21,439.0 | 8,656.8 | Municipal waste | Continued decline in total disposed waste |
| 2023 | 19,916.5 | 8,363.2 | Municipal waste | Lowest quantity after 2020 peak; gradual stabilization |

The composition of the disposed waste demonstrates that biodegradable waste consistently represents the largest fraction of the municipal waste stream. This finding indicates a significant opportunity for introducing or expanding biological treatment methods such as composting, which could substantially reduce the volume of waste requiring final disposal. Plastic packaging also represents a considerable proportion of the waste stream, emphasizing the need for more efficient source separation and recycling systems. The presence of potentially hazardous household waste, including batteries and electronic components, further underlines the necessity for separate collection schemes to prevent soil and groundwater contamination.

The assessment of pre-disposal treatment confirms that the current system primarily relies on mechanical separation, compaction, and periodic covering of waste. These measures contribute to improved landfill operation and more efficient use of available disposal space. However, the absence of fully developed biological and chemical treatment technologies limits the overall efficiency of waste recovery and the transition toward a circular economy. The analysis of industrial waste generators reveals that only a limited number of companies are responsible for the majority of industrial waste delivered to the landfill. Companies such as Alfaplam, Geox, Simpo, and Yumco consistently represent the largest contributors during the analyzed period, while several smaller companies generate comparatively low quantities of waste. This concentration of waste generation suggests that targeted waste prevention programs and cleaner production measures implemented within major industrial facilities could significantly reduce the overall amount of industrial waste requiring disposal.

The results also indicate that recyclable materials are not equally represented in the recovery process. While PET packaging and cardboard are collected in measurable quantities, the complete absence of recorded deliveries of metal and glass during certain periods suggests deficiencies in the collection system or alternative waste management routes that require further investigation. Strengthening selective collection and improving recycling infrastructure would increase material recovery rates and reduce dependence on landfill disposal.

Overall, the obtained results demonstrate that the "Meteris" landfill performs an important function within the regional waste management system by ensuring controlled disposal and basic waste treatment. However, the findings also indicate considerable opportunities for improving recycling efficiency, biological waste treatment, hazardous waste separation, and circular economy practices. Such improvements would contribute simultaneously to environmental protection, resource conservation, and the long-term economic sustainability of the waste management system.

# 4. DISCUSSION

The present study examined the economic and environmental aspects of industrial waste management through a case study of the Meteris regional landfill in Vranje, Serbia. The findings demonstrate that although measurable progress has been achieved in landfill operation and waste handling procedures, the current waste management system remains predominantly disposal-oriented, indicating that significant improvements are still required to achieve the objectives of sustainable waste management and the circular economy [8][9].

The analysis of operational data collected during the period 2018–2023 revealed that municipal waste continues to dominate the total waste stream, while industrial waste accounts for a considerably smaller proportion of the total disposed mass. Nevertheless, industrial waste requires particular attention because of its heterogeneous composition and the potential presence of hazardous substances capable of contaminating soil, groundwater, and surface waters if not managed appropriately. Similar observations have been reported in previous studies [10], which emphasize that environmental risks associated with industrial waste are determined not only by quantity but also by waste composition and hazardous characteristics [11].

One of the most significant findings of this study is the continued reliance on landfilling as the primary waste management practice. Although mechanical compaction, periodic soil covering, and partial waste separation are routinely implemented at the Meteris landfill, advanced treatment technologies remain largely absent. According to the waste hierarchy established by the European Union Waste Framework Directive (Directive 2008/98/EC) [12], landfilling should represent the least preferable management option after waste prevention, reuse, recycling, and recovery. Consequently, the current management approach demonstrates only partial alignment with internationally accepted principles of sustainable waste management.

The analysis further indicates that biodegradable waste constitutes the largest fraction of disposed waste throughout the analysed period. From the perspective of the circular economy, this represents a substantial untapped resource. Instead of final disposal, biodegradable waste could be diverted toward composting or anaerobic digestion, producing compost, biogas, and renewable energy while simultaneously reducing methane emissions from landfill decomposition. Numerous studies have demonstrated that biological treatment significantly decreases greenhouse gas emissions and extends landfill lifespan by reducing the quantity of biodegradable material requiring final disposal [13].

The relatively low recovery of recyclable materials observed in this study also highlights important opportunities for improvement. PET packaging and cardboard represent the principal recyclable fractions recovered at the landfill, whereas recovery of metals and glass remains extremely limited or absent during several reporting periods. These findings suggest deficiencies in source separation systems, collection infrastructure, or market conditions for recyclable materials. International experience consistently demonstrates that effective source separation, combined with public awareness campaigns and extended producer responsibility (EPR) schemes, substantially increases recycling rates and reduces landfill dependency.

From an economic perspective, the results indicate that waste management should not be regarded solely as an environmental obligation but also as a strategic economic activity [14]. Disposal costs continue to increase because of landfill operation, environmental monitoring, equipment maintenance, and future remediation liabilities. Conversely, investments in recycling infrastructure and material recovery technologies have the potential to generate secondary raw materials, reduce raw material consumption, create employment opportunities, and improve regional economic competitiveness. These observations are fully consistent with circular economy principles, which consider waste as a valuable economic resource rather than an unavoidable by-product of production.

The identification of several industrial companies as major waste generators provides valuable information for environmental planning and policy development. However, large waste quantities should not automatically be interpreted as evidence of poor environmental performance, since they may primarily reflect production scale. Future assessments should therefore incorporate waste generation intensity indicators (e.g., kilograms of waste per unit of production or per employee), which would enable more meaningful comparisons among industrial sectors.

The study also highlights several institutional challenges. Although Serbia has significantly advanced the harmonization of its environmental legislation [15] with European Union requirements [16], implementation remains constrained by limited financial resources, insufficient inspection capacity, and uneven enforcement of regulatory provisions. Similar implementation gaps have been documented throughout many transition economies [17], where legislative reforms frequently progress faster than infrastructure development and institutional capacity building [18].

Overall, the obtained results confirm that future improvements should prioritize the development of integrated waste management systems based on the principles of prevention, recycling, resource recovery, and continuous environmental monitoring. Such an approach would simultaneously reduce environmental pressures, improve economic efficiency, and facilitate Serbia's alignment with European environmental policies and circular economy objectives [19][20].

The case study of waste disposal at the Meteris landfill in Vranje is similar to many waste disposal solutions in Serbian cities. However, there are also cities that do not have landfills at all. The topic of selecting and depositing waste in Serbia is a great challenge and action for researchers and engineers.

# 5. CONCLUSIONS

This study evaluated the economic and environmental aspects of industrial waste management through a comprehensive case study of the Meteris regional landfill in Vranje, Serbia. By integrating operational data collected during the period 2018–2023 with an assessment of waste management practices and the applicable legislative framework, the research provides a comprehensive overview of current landfill performance and identifies opportunities for further improvement.

The results demonstrate that landfill disposal remains the dominant waste management option within the analysed region. Although operational procedures, including waste compaction, partial waste separation, and routine environmental protection measures, contribute to more efficient landfill management, recycling and resource recovery activities remain insufficiently developed. Consequently, considerable quantities of potentially recyclable and biodegradable materials continue to be disposed of through landfilling rather than being reintegrated into the production cycle.

From an environmental perspective, the continued disposal of mixed waste increases the risk of long-term soil and groundwater contamination, greenhouse gas emissions, and inefficient resource utilization. The predominance of biodegradable waste further emphasizes the necessity of introducing biological treatment technologies capable of reducing methane emissions while simultaneously generating valuable secondary products.

The economic analysis indicates that investments in modern waste treatment technologies should be regarded as long-term investments rather than operational expenses. Expanding recycling infrastructure, improving waste separation systems, and implementing material recovery technologies would reduce landfill operating costs, extend landfill lifespan, and create additional economic value through the production of secondary raw materials.

The study further demonstrates that achieving sustainable waste management requires stronger cooperation among local governments, industrial stakeholders, environmental authorities, and waste management operators. Effective implementation of environmental legislation should be supported through continuous monitoring, improved inspection mechanisms, increased institutional capacity, and stronger incentives encouraging waste prevention and recycling.

The scientific contribution of this research lies in presenting an integrated evaluation of operational, environmental, and economic aspects of industrial waste management using a regional landfill as a representative case study. The proposed analytical framework may also be applied to similar landfills operating under comparable technical and institutional conditions in other transition economies.

Several research limitations should be acknowledged. The analysis relied primarily on operational landfill records and did not include direct measurements of environmental quality parameters such as soil contamination, groundwater quality, landfill gas emissions, or leachate composition. Moreover, the findings are based on a single case study and therefore cannot be generalized to all landfill systems without additional comparative investigations.

Future research should focus on quantitative environmental risk assessment, life cycle assessment (LCA) of alternative waste management scenarios, cost-benefit analyses of recycling investments, and the application of digital technologies for intelligent waste management. Furthermore, comparative studies involving multiple regional landfills would provide a broader understanding of waste management performance and facilitate the development of evidence-based national waste management strategies.

Overall, the findings confirm that the transition from conventional landfill-based waste management toward a circular economy model represents both an environmental necessity and an economic opportunity. Such a transition would contribute not only to environmental protection but also to improved resource efficiency, enhanced industrial sustainability, and long-term regional development.

# ACKNOWLEDGEMENT

# The results presented here are supported by the Мinistry of science, technological development and innovation of the Rеpublic of Serbia under contract 451-03-33/2026-03/200213 dated 05.02.2026. Year.

REFERENCES

- Mohideen, A. H. (2026). Multidisciplinary Perspectives on the Sustainable Development Goals: Special Issue on Sustainable Development Goals (SDGs) and Economic Development-JMCECOSDG 2026. Jamal Academic Research Journal: An Interdisciplinary, 7(3).

- Mesjasz-Lech, A. (2024). Trends of industrial waste generation in manufacturing enterprises in the context of waste prevention—Shift-share analysis for European Union countries. Sustainability, 17(1), 34.

- Spani, R. C. (2020). The new circular economy action plan. FEEM policy brief, (09-2020).

- Directive, E. U. (2008). 98/EC on waste (Waste Framework Directive). Directive, S. F. (2008). Directive 2008/56/EC of the European Parliament and of the Council. Journal). Council Decision of, 17.

- Data of public utility company (PUC) "Meteris", landfill in Vranje, 2018 - 2024 (in Serbian)

- Environmental Protection Agency. (2020). Report on the state of the environment in the Republic of Serbia for 2019. Belgrade: SEPA (in Serbian)

- PUC "Meteris" - Internal documentation and annual reports on the operation of the landfill, Vranje (in Serbian)

- Liu, M., & Zhang, X. (2026). Waste not, emit not: How circular economy strategies shape climate and growth outcomes in G-20 nations? Journal of Cleaner Production, 540, 147415.

- Macarthur, E. L. L. E. N., & Heading, H. E. A. D. I. N. G. (2019). How the circular economy tackles climate change. Ellen MacArthur Found, 1(3), 1-71.

- Kaza, S., Yao, L., Bhada-Tata, P., & Van Woerden, F. (2018). What a waste 2.0: a global snapshot of solid waste management to 2050. World Bank Publications.

- Salvetti, M. (2025). Municipal waste management in the European Union. ECONOMIA PUBBLICA, (2025/2).

- Gharfalkar, M., Court, R., Campbell, C., Ali, Z., & Hillier, G. (2015). Analysis of waste hierarchy in the European waste directive 2008/98/EC. Waste management, 39, 305-313.

- Kumar, A. (2024). United nations environment programme (UNEP). Yearbook of International Environmental Law, 35(1), yvaf054.

- Chioatto, E., & Sospiro, P. (2023). Transition from waste management to circular economy: the European Union roadmap. Environment, Development and Sustainability, 25(1), 249-276.

- Law on Waste Management, "Official Gazette of RS", no. 36/2009, 88/2010, 14/2016 and 95/2018. (in Serbian)

- Halkos, G. E., & Aslanidis, P. S. C. (2024). Reviewing the integrated institutional waste-related framework for circular economy in the European Union. Waste Management Bulletin, 2(3), 28-35.

- Mihajlović, V., Vujić, G., Stanisavljević, N., & Batinić, B. (2016). Financial implications of compliance with EU waste management goals: Feasibility and consequences in a transition country. Waste Management & Research, 34(9), 923-932.

- Maletz, R., Wohmann, S., & Pukhnyuk, A. (2018). Comparison of waste management systems in western and transition economies within the WaTra project. Detritus, 2(1), 96.

- Vujic, G., Stanisavljevic, N., Batinic, B., Jurakic, Z., & Ubavin, D. (2017). Barriers for implementation of “waste to energy” in developing and transition countries: a case study of Serbia. Journal of Material Cycles and Waste Management, 19(1), 55-69.

- Čepić, Z., Bošković, G., Ubavin, D., & Batinić, B. (2022). Waste-to-Energy in Transition Countries: Case Study of Belgrade (Serbia). Polish Journal of Environmental Studies, 31(5).

EKONOMSKO EKOLOŠKI ASPEKTI UPRAVLJANJA OTPADOM - PRIMER DEPONIJE „METERIS“

## Apstrakt: Upravljanje industrijskim otpadom predstavlja jedan od ključnih izazova savremenih sistema zaštite životne sredine zbog njegovog potencijalnog uticaja na kvalitet zemljišta, voda, vazduha i zdravlje stanovništva. Ovaj rad analizira ekonomske i ekološke aspekte upravljanja otpadom na primeru regionalne deponije „Meteris“ u Vranju. Istraživanje je zasnovano na analizi dostupne dokumentacije, statističkih podataka o količinama odloženog otpada u periodu 2018–2023. godine, kao i relevantnog zakonodavnog i institucionalnog okvira. Posebna pažnja posvećena je identifikaciji izazova u upravljanju industrijskim otpadom, uključujući ograničene kapacitete za selekciju i tretman otpada, nedovoljnu primenu principa cirkularne ekonomije i potrebu za unapređenjem institucionalne koordinacije. Rezultati ukazuju da, uprkos određenim unapređenjima u organizaciji rada deponije, dominantan način upravljanja otpadom i dalje predstavlja odlaganje, dok je iskorišćenost reciklaže i drugih metoda oporavka otpada nedovoljna. Dobijeni rezultati potvrđuju potrebu za modernizacijom sistema upravljanja otpadom kroz razvoj infrastrukture za separaciju i reciklažu, unapređenje monitoringa životne sredine i dosledniju primenu zakonskih propisa. Predložene mere mogu doprineti smanjenju negativnih uticaja na životnu sredinu i efikasnijem korišćenju resursa u skladu sa principima održivog razvoja i cirkularne ekonomije.

Ključne reči: industrijski otpadne materije, upravljanje otpadom, deponija Meteris, cirkularna ekonomija, zaštita životne sredine, održivi razvoj.